| Title | Prepare electrical machines for rewinding |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 8 |

| Purpose | This unit standard is intended for people who have responsibility for preparing machines for rewinding. People credited with this unit standard are able to: – dismantle and inspect electrical machines in accordance with industry practice; – strip out windings in accordance with industry practice; – refurbish casing in accordance with industry practice; and – prepare armature/rotor and/or yoke in accordance with industry practice. |
| --- | --- |

| Classification | Electrical Engineering > Electrical Machines |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance information

1	This unit standard has been developed for learning and assessment on-job.

2	Definitions

Electrical machines – are defined as electric motors, generators, alternators, and transformers.

Industry practice – those practices that competent practitioners within the industry recognise as current industry best practice.

3	Assessment

a	The machines chosen are left to the discretion of the assessor, but must be sufficient to assess competence in all outcomes of the standard.

b	Performance in relation to the outcomes of this unit standard must comply with the Health and Safety at Work Act 2015, associated regulations, and any applicable company safety and health procedures.

Outcomes and performance criteria

Outcome 1

Dismantle and inspect electrical machines in accordance with industry practice.

Performance criteria

1.1	Record position of components.

1.2	Check air gap for uniformity.

1.3	Identify cause of a non-uniform air gap.

Range	typical causes may include but are not limited to – worn or misaligned bearings, bearing journals, or housings; bent shaft.

1.4	Clean components and, where appropriate, degrease with suitable solvents.

1.5	Carry out visual inspection of all components to assess wear and deterioration.

Range	may include but is not limited to –

for wear – bearings, brushgear, commutator, slip-rings, centrifugal switches, rotor and stator surfaces;

for deterioration – insulation, seals, gaskets, air passages, terminations, fasteners, casings, capacitors.

Outcome 2

Strip out windings in accordance with industry practice.

Performance criteria

2.1	Collect and record winding data.

Range	coils per slot, coil turns and polarities; size of conductor; insulation types, thickness and layers; coil dimensions and shapes; connections.

2.2	Identify tools and equipment to match the type of windings.

Range	may include but is not limited to – cutting and stripping tools, warming and pulling equipment; burnout oven.

2.3	Carry out cutting and stripping methods to match the machine.

Range	generators, motors, transformers.

2.4	Clean and check winding slots for damage.

2.5	Test core in accordance with industry practice.

Outcome 3

Refurbish casing in accordance with industry practice.

Performance criteria

3.1	Strip and clean casing.

3.2	Inspect casing for warping or dents and repair where required.

3.3	Mask and coat casing.

Range	may include but is not limited to – powder coat, spray paint, hand paint.

Outcome 4

Prepare armature/rotor and/or yoke in accordance with industry practice.

Performance criteria

4.1	Clean armature/rotor and/or yoke and prepare for windings.

4.2	Inspect armature/rotor and/or yoke for damage or wear.

| Planned review date | 31 December 2023 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 29 April 1994 | 31 December 2013 |
| Review | 2 | 23 April 1996 | 31 December 2013 |
| Revision | 3 | 3 April 2001 | 31 December 2013 |
| Review | 4 | 26 August 2002 | 31 December 2013 |
| Review | 5 | 22 August 2008 | 31 December 2023 |
| Rollover and Revision | 6 | 15 March 2012 | 31 December 2023 |
| Revision | 7 | 15 January 2014 | 31 December 2023 |
| Review | 8 | 22 August 2019 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0003 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact The Skills Organisation reviewcomments@skills.org.nz if you wish to suggest changes to the content of this unit standard.