| Title | Select, use, and care for advanced engineering measuring equipment |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 3 |

| Purpose | People credited with this unit standard are able to select, use, and care for advanced engineering measuring equipment. |
| --- | --- |

| Classification | Mechanical Engineering > Engineering - Measurement |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Definitions

Accepted industry practice – approved codes of practice and standardised procedures accepted by the wider mechanical engineering industry sectors as examples of best practice.

Digital level indicator – digital spirit level with laser beam such as used in construction industry and for alignment of machinery, or a device measuring tilt on two axis’ to determine the levelness of a platform.

Displacement transducer – transducer to measure linear displacement, and connected to a control system.

2	Recommended for entry

Unit 4435, Select, use, and care for engineering dimensional measuring equipment.

Outcomes and performance criteria

Outcome 1

Select advanced engineering measuring equipment.

Range	examples of advanced engineering measuring equipment are – gauge blocks, length bars, cylindrical squares, sine bars, surface texture standard blocks, digital level indicator, displacement transducer, precision squares, precision surface tables;

evidence is required for gauge blocks, and at least three other items.

Performance criteria

1.1	Measuring equipment is selected having regard to the quantity to be measured, expected magnitude, ability to meet required tolerances, and effects of temperature and humidity.

1.2	Measurement technique is planned and explained prior to commencing work.

1.3	Measuring equipment is inspected for damage and faults and supervisor advised.

Range	damage and faults may include but are not limited to – deformation, scratches and burrs, breakages, missing parts, not zeroing, expired calibration.

Outcome 2

Use advanced engineering measuring equipment.

Range	items selected from outcome 1.

Performance criteria

2.1	Use of the measuring equipment achieves the required accuracy, and the tolerance of the measurement is quoted.

2.2	Measurements are validated by re-measurement with the same or alternate equipment.

2.3	Measuring equipment and objects to be measured are not damaged during measurement.

2.4	Measurements are recorded in accordance with accepted industry practice.

Range	number of decimal places, units, prefixes, symbols, accuracy.

Outcome 3

Care for advanced engineering measuring equipment.

Range	items selected from outcome 1.

Performance criteria

3.1	Measuring equipment is handled and stored in a manner that maintains its integrity.

| Planned review date | 31 December 2022 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 23 May 1995 | 31 December 2011 |
| Revision | 2 | 14 April 1997 | 31 December 2011 |
| Revision | 3 | 5 January 1999 | 31 December 2011 |
| Revision | 4 | 23 May 2001 | 31 December 2011 |
| Review | 5 | 21 February 2005 | 31 December 2014 |
| Review | 6 | 17 June 2011 | 31 December 2022 |
| Review | 7 | 17 August 2017 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0013 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Competenz qualifications@competenz.org.nz if you wish to suggest changes to the content of this unit standard.