| 40447 | Operate geothermal steam-field processes and equipment in an energy and chemical plant |
| --- | --- |

| Kaupae ¦ Level | 4 |
| --- | --- |
| Whiwhinga ¦ Credit | 20 |
| Whāinga ¦ Purpose | This skill standard is intended for people working as energy and chemical process operators in an energy and chemical plant. People credited with this skill standard are able to operate geothermal steam-field processes and equipment in an energy and chemical plant. This skill standard can be used in the New Zealand Energy and Chemical qualifications at Level 4 and above. |

Hua o te ako me Paearu aromatawai | Learning outcomes and assessment criteria

| Hua o te ako ¦ Learning outcomes | Paearu aromatawai ¦ Assessment criteria |
| --- | --- |
| - Operate geothermal steam-field processes and equipment in an energy and chemical plant. | - Identify hazards associated with working in a geothermal steam-field operation and apply steps to minimise the risk. |
|  | - Carry out procedures for entering the geothermal steam-field in accordance with organisational requirements. |
|  | - Identify consents associated with the operation of the geothermal steam-field in terms of their requirements for a site. |
|  | - Carry out well operating procedures. |
|  | - Carry out plant equipment operating procedures. |
|  | - Carry out routine inspections, testing and monitoring. |
|  | - Identify geothermal steam-field equipment faults and take corrective actions in accordance with organisational requirements. |

Pārongo aromatawai me te taumata paearu | Assessment information and grade criteria

Assessment specifications:

- evidence for this skill standard must be supplied from the workplace.

- evidence must be presented in accordance with organisational requirements.

- 1a: evidence of a minimum of five (5) hazards is required.

- 1d: evidence of at least five (5) is required.

- 1e: plant equipment – evidence of at least five (5) is required; plant equipment procedures – evidence of at least five (5) is required.

- 1g: evidence of fault identification and corrective actions must be documented in accordance with organisational requirements; evidence of at least five (5) faults is required.

Definitions:

Energy and chemical plant may be in – petrochemical, agri-nutrient, power generation, dairy processing, meat processing, and wood fibre manufacturing, or other plants that operate with a combination of high temperatures, pressures, steam and/or chemicals in gas, liquid or solid form.

Organisational requirements – documented policies and procedures. These may include: equipment manufacturers’ procedures; plant procedures; suppliers’ instructions; site signage; codes of practice; company health and safety plans; on site briefings; and supervisor’s instructions. This includes all regulatory and legislative obligations that apply to the plant.

Plant – the operational unit, equipment and/or workplace at which the person is working.

Ngā momo whiwhinga | Grades available

Achieved

Ihirangi waitohu | Indicative content

- Consent requirements such as – total production, reinjection, discharges to air, water and ground, noise.

- Well operating procedures such as – vent gas cap, warm up, place on bleed, bring on-line, shut in, isolate, de-isolate down-hole testing.

- Plant equipment such as – steam field pipeline/production/ reinjection/protection, separator, scrubber, pumps, dump station, vent station.

- Plant equipment operating procedures such as – de-isolate, warm up, bring on-line, take out of service, isolate, test protection and protective devices, cross country pipeline warming, pipelines with low points, precautions against water hammer.

Rauemi | Resources

Legislation relevant to this skill standard includes but is not limited to:

- Health and Safety at Work Act 2015;

- Hazardous Substances and New Organisms Act 1996;

- Resource Management Act 1991;

- and any subsequent amendments.

Pārongo Whakaū Kounga | Quality assurance information

| Ngā rōpū whakatau-paerewa ¦ Standard Setting Body | Hanga-Aro-Rau Manufacturing, Engineering and Logistics Workforce Development Council |
| --- | --- |
| Whakaritenga Rārangi Paetae Aromatawai ¦ DASS classification | Manufacturing > Energy and Chemical Plant > Operation of Energy and Chemical Plant |
| Ko te tohutoro ki ngā Whakaritenga i te Whakamanatanga me te Whakaōritenga ¦ CMR | 0079 |

| Hātepe ¦ Process | Putanga ¦ Version | Rā whakaputa ¦ Review Date | Rā whakamutunga mō te aromatawai ¦ Last date for assessment |
| --- | --- | --- | --- |
| Rēhitatanga ¦ Registration | 1 | 24 April 2025 | N/A |
| Kōrero whakakapinga ¦ Replacement information | This skill standard replaced unit standard 28160. |  |  |
| Rā arotake ¦ Planned review date | 31 December 2029 |  |  |

Please contact Hanga-Aro-Rau Manufacturing, Engineering and Logistics Workforce Development Council at qualifications@hangaarorau.nz to suggest changes to the content of this skill standard.