| 40744 | Operate rendering equipment and associated processes |
| --- | --- |

| Kaupae ¦ Level | 4 |
| --- | --- |
| Whiwhinga ¦ Credit | 20 |
| Whāinga ¦ Purpose | This skill standard is for people working in a meat processing operation. It recognises the skill required to operate rendering equipment, including monitoring equipment and implementing corrective procedures for process variations. It can be used in the New Zealand meat processing qualifications at Level 4 and above. |

Hua o te ako me Paearu aromatawai | Learning outcomes and assessment criteria

| Hua o te ako ¦ Learning outcomes | Paearu aromatawai ¦ Assessment criteria |
| --- | --- |
| - Start up a rendering operation. | - Prepare, blend and load raw product into holding receptacles in accordance with legislative requirements. |
|  | - Conduct preoperational safety checks and prepare rendering equipment in accordance with legislative requirements. |
|  | - Monitor control panel meters, gauges, and mimic lights during start-up and make adjustments to machine operations in accordance with manufacturer’s specifications. |
|  | - Identify and rectify any malfunctions that occur during start-up in accordance with manufacturer’s specifications. |
| - Operate monitoring equipment and implement corrective procedures for process variations associated with a rendering operation. | - Switch automatic systems, where used, from manual to automatic when machinery is fully operational and control panel readings reach specified levels in accordance with manufacturer’s specifications. |
|  | - Separate all stages in the handling, storage, and processing of edible and inedible rendered products. |
|  | - Monitor control panel and operate equipment in accordance with manufacturer’s specifications. |
|  | - Take, test and record product sample levels in accordance with legislative requirements. |
|  | - Identify, rectify and record any malfunctions during operations. |
|  | - Handle spills and rejected products and blockages to process operations. |
|  | - Dispose of effluent from rendering operations within tolerance levels. |
|  | - Monitor non condensed emissions and vapours levels in accordance with legislative requirements. |
| - Shut down a rendering operation. | - Switch automatic systems, where used, to manual operation in accordance with manufacturer’s specifications. |
|  | - Shut down sections of rendering systems in sequence as product throughput is completed. |
|  | - Blow and clean tallow lines before system shut down. |
|  | - Ensure power, steam, and water supplies remain available for maintenance and cleaning, in accordance with manufacturer’s specifications. |
|  | - Carry out isolation and lock out procedures for hygienic cleaning and servicing. |

Pārongo aromatawai me te taumata paearu | Assessment information and grade criteria

Assessment specifications:

- All assessment activities must be carried out in accordance with organisational requirements.

- Rendering operation may be either a low or high temperature rendering operation.

- Evidence of five preoperational safety checks is required.

- Evidence of four monitoring activities is required.

Definitions:

FFA levels – free fatty acid levels.

Manufacturer’s specifications – instructions from the manufacturer which outline the specifications, parts, maintenance, service schedule, repair and running of specific equipment.

Organisational requirements – instructions to staff on policies and procedures that are documented in memo, electronic, or manual format and available in the workplace, and are consistent with manufacturer’s requirements.

Ngā momo whiwhinga | Grades available

Achieved

Ihirangi waitohu | Indicative content

Preoperational safety checks

- adherence to machine isolation procedures.

- visual and auditory checks.

- verification of equipment functionality such as lock out tags, bungs, lids, covers, safety guards, alarm sensors, driers, blood processing and size reduction equipment, re-cycle tallow tanks, liquid phase tanks, separators, decanters, conveyors, render vessels, metal detectors, and control panel.

Monitoring activities

- fault lights, temperature gauges, pH readings, amp meter readings, particle size, FFA levels, flow rates, control panel readings.

- adjusting operations to keep the rendering process within tolerance.

- testing and recording product sample levels.

- visual and auditory checks during operation.

Rauemi | Resources

- Health and Safety at Work Act 2015.

- Animal Products Act 1999.

- Resource Management Act 1991.

- Ministry for Primary Industries: Code of Practice Red Meat: Rendering (NZFSA, 2009) available at: https://www.mpi.govt.nz/dmsdocument/1366-rendering-code-of-practice.

Pārongo Whakaū Kounga | Quality assurance information

| Ngā rōpū whakatau-paerewa ¦ Standard Setting Body | Hanga-Aro-Rau Manufacturing, Engineering and Logistics Workforce Development Council |
| --- | --- |
| Whakaritenga Rārangi Paetae Aromatawai ¦ DASS classification | Manufacturing > Meat Processing > Meat Industry - Rendering |
| Ko te tohutoro ki ngā Whakaritenga i te Whakamanatanga me te Whakaōritenga ¦ CMR | 0033 |

| Hātepe ¦ Process | Putanga ¦ Version | Rā whakaputa ¦ Review Date | Rā whakamutunga mō te aromatawai ¦ Last date for assessment |
| --- | --- | --- | --- |
| Rēhitatanga ¦ Registration | 1 | 28 August 2025 | N/A |
| Kōrero whakakapinga ¦ Replacement information | This skill standard replaced unit standard 31948. |  |  |
| Rā arotake ¦ Planned review date | 31 December 2030 |  |  |

Please contact Hanga-Aro-Rau Manufacturing, Engineering and Logistics Workforce Development Council at qualifications@hangaarorau.nz to suggest changes to the content of this skill standard.