| Title | Apply liquid roof membrane |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 30 |

| Purpose | This unit standard is intended for use in the training and assessment of people carrying out the installation of liquid roof membrane systems. People credited with this unit standard are able to: - confirm materials and availability of equipment for the application of liquid roof membrane; - complete preparations for the application of liquid roof membrane; and - apply, complete, and finish liquid roof membrane installation. |
| --- | --- |

| Classification | Plumbing, Gasfitting and Drainlaying > Roofing |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	This unit standard has been developed for learning and assessment on-job.

2	References

Construction Contracts Act 2002;

Historic Places Act 1993;

Local Government Act 2002;

Building Act 2004 and associated regulations;

Health and Safety at Work Act 2015;

Resource Management Act 1991;

Acceptable Solutions and Verification Methods for New Zealand Building Code Clause E2 External Moisture;

Licensed Building Practitioner Scheme – Understanding the Regulatory Environment Booklet – available at: http://www.business.govt.nz/lbp/the-board/getting-licensed/applying-for-licensing;

NZS 3604:2011 Timber-framed buildings;

NZS 3902:2004 Housing, alterations and small buildings contract;

NZS 4121:2001, Design for access and mobility: Buildings and associated facilities;

New Zealand Building Code Hand Book – available at: http://www.building.govt.nz/building-code-compliance/building-code-and-handbooks/building-code-handbook;

and all subsequent amendments and replacements.

3	This unit standard applies to both new roof or re-roofing installations.

4	Range

a	Candidates must refer to current legislation and Standards during assessment.

b	Demonstration of safe working practices are essential components of assessment of this unit standard.

c	All activities and evidence presented for all outcomes and performance criteria in this unit standard must be in accordance with:

i	legislation;

ii	policies and procedures;

iii	ethical codes;

iv	Standards – may include but are not limited to those cited in the New Zealand Building Code Hand Book;

v	applicable site, enterprise, and industry practice; and,

vi	manufacturers’ instructions, specifications, and data sheets.

Outcomes and performance criteria

Outcome 1

Confirm materials and availability of equipment for the application of liquid roof membrane.

Performance criteria

1.1	Assess the job and draw up a plan of work.

Range	includes but is not limited to – task analysis, checklists.

1.2	Confirm that the materials for the job are consistent with the job and manufacturers' specifications.

Range	includes but is not limited to – membrane, cladding, flashings, sealants, tapes, cleaners, fillers.

1.3	Confirm equipment availability.

Range	includes but is not limited to – fire extinguisher, scaffolding, ladders, tools, brushes, belt sander, airless spray units, propane cylinder and torch, angle grinder, heat gun, saws, router, pop riveter, rollers.

Outcome 2

Complete preparations for the application of liquid roof membrane.

Performance criteria

2.1	Install fire extinguishers on site and notify workers location, extinguisher type, and extinguisher capabilities.

2.2	Prepare area in accordance with site requirements.

Range	area cleaned with blower or vacuum cleaner, moisture content of substrate within accepted parameters, ambient temperature within manufacturers' instructions.

2.3	Identify any defects in substrate and rectify and rout lap rebates where required.

2.4	Install roof vents where required.

Outcome 3

Apply, complete, and finish liquid roof membrane installation.

Performance criteria

3.1	Install membrane sequentially.

Range	primer where required, grid layout set, corner under tape applied, sheets positioned and spot tacked, material folded back in sequence to mask laps.

3.2	Roll out membrane to remove air.

3.3	Apply membrane over flashings.

3.4	Correct any defects.

3.5	Remove surplus material and leave roof and site in a tidy and safe condition that allows any further work to be carried out safely.

| Replacement information | This unit standard was replaced by skill standard 40200. |
| --- | --- |

This unit standard is expiring.  Assessment against the standard must take place by the last date for assessment set out below.

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 21 May 1997 | 31 December 2020 |
| Review | 2 | 27 October 2006 | 31 December 2020 |
| Review | 3 | 23 January 2009 | 31 December 2020 |
| Review | 4 | 16 March 2017 | 31 December 2027 |
| Rollover | 5 | 25 August 2022 | 31 December 2027 |
| Review | 6 | 28 November 2024 | 31 December 2027 |

| Consent and Moderation Requirements (CMR) reference | 0003 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.