| Title | Perform hydrostatic tests on gas mains and services |  |  |
| --- | --- | --- | --- |
| Level | 5 | Credits | 10 |

| Purpose | People credited with this unit standard are able to: - demonstrate knowledge of company procedures, documentation, hazards, equipment, and fittings for hydrostatic tests on gas mains and services; - prepare a hydrostatic test plan; - prepare for hydrostatic testing on gas mains and services; - carry out a hydrostatic testing pressure test; and - complete reporting and documentation procedures. |
| --- | --- |

| Classification | Gas Industry > Gas Network Construction |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Evidence presented for assessment against this unit standard must be consistent with safe working practices and be in accordance with applicable manufacturer specifications, company procedures, and legislative requirements.

2	Legislation, standards, and guidelines relevant to this unit standard include:

- Health and Safety at Work Act 2015;

- Resource Management Act 1991;

- AS/NZS 4645.1: 2018 Gas distribution networks – Part 1: Network management;

- AS/NZS 4645.2: 2018 Gas distribution networks – Part 2: Steel pipe systems;

- AS/NZS 4645.3: 2018 Gas distribution networks – Part 3: Plastic pipe systems;

- AS 2885.1: 2018 Pipelines – Gas and liquid petroleum Design and Construction;

- AS 2885.3: 2022 Pipelines – Gas and liquid petroleum, Part 3: Operation and Maintenance; available from www.standards.org.au;

- Excavation Safety: Good Practice Guidelines, July 2016; available from www.worksafe.govt.nz.

3	Assessment

It is recommended people achieve Unit 32339, Internally clean a gas pipe in a gas network, or demonstrate equivalent knowledge and skills before being assessed against this unit standard.

4	Definition

Company procedures refer to the documented methods for performing work activities, and include health and safety, operational, environmental, and quality management requirements.  They may refer to legislation, regulations, guidelines, standard operating procedures, manuals, codes of practice, or policy statements.

Outcomes and performance criteria

Outcome 1

Demonstrate knowledge of company procedures, documentation, hazards, equipment, and fittings for hydrostatic tests on gas mains and services.

Performance criteria

1.1	Documentation and company procedures for hydrostatic testing are interpreted and described in terms of purpose.

Range	may include – network technical standard, standard operating procedure, safe work procedure, work instruction, job hazard analysis, environmental plan;

evidence of three items of documentation is required.

1.2		Job documentation and instructions are confirmed for accuracy.

Range	as-built drawings, waste disposal instructions, test certificates.

1.3		Potential environmental and safety hazards are described in terms of their controls.

Range	work in excavations, other utilities, stored energy, uncontrolled release of test water, traffic, public.

1.4		Hydrostatic testing is described.

Range	test objectives, test methods, test pressure, test lengths, test procedure.

1.5		Hydrostatic testing equipment, components and materials are described in terms of their function.

Range 	pigs, pig trap, pig catcher, test pump, test manifold, calibrated test gauge, drying medium, test water.

1.6		Factors that affect hydrostatic test results are described in terms of impact and controls.

Range	elevation, temperature.

Outcome 2

Prepare a hydrostatic test plan.

Performance criteria

2.1	A hydrostatic test plan is prepared for review and approval.

Outcome 3

Prepare for hydrostatic testing on gas mains and services.

Performance criteria

3.1	Safety and environmental risks are identified and controlled.

3.2 	Hydrostatic test equipment is prepared, positioned, and connected.

3.3 	Preparation ensures that pipe work to be tested and test fittings meet specifications and are anchored in position.

3.4	Air vents and drainage points are positioned.

3.5 	Test water supply is connected and made ready for the test.

Outcome 4

Carry out a hydrostatic testing pressure test.

Performance criteria

4.1	Hydrostatic test water is injected to the pipe work in accordance with the hydrostatic test plan.

4.2	Hydrostatic test pressure is raised, and pressure level and security of pipe work and test equipment are monitored.

4.3	Test equipment is checked for leaks in accordance with the hydrostatic test plan.

4.4	Stabilisation period is timed in accordance with the hydrostatic test plan.

4.5	Pressure test readings are obtained in accordance with the hydrostatic test plan.

4.6	Test period is applied, and test readings are recorded.

4.7	Where test results show a pressure loss, leaks are traced and repaired, and retesting is completed.

4.8	Pipe is depressurised on test completion in a controlled manner and test equipment is disconnected.

4.9	Test water is disposed of.

4.10	Pipe is internally cleaned and dried and made ready for commissioning in accordance with industry standards.

Outcome 5

Complete reporting and documentation procedures.

Performance criteria

5.1	Recording and documentation procedures are completed, and information is communicated to internal and external parties as required.

Range 	may include – job card, test results, consents, special conditions, completion notice, additional work;

evidence of three procedures is required.

| Planned review date | 31 December 2027 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 25 June 1997 | 31 December 2018 |
| Revision | 2 | 3 August 2000 | 31 December 2018 |
| Review | 3 | 22 October 2002 | 31 December 2018 |
| Review | 4 | 20 November 2006 | 31 December 2018 |
| Review | 5 | 21 May 2010 | 31 December 2020 |
| Review | 6 | 17 August 2017 | 31 December 2025 |
| Revision | 7 | 30 August 2018 | 31 December 2025 |
| Review | 8 | 24 November 2022 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0014 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Waihanga Ara Rau Construction and Infrastructure Workforce Development Council at qualifications@WaihangaAraRau.nz if you wish to suggest changes to the content of this unit standard.