| Title | Demonstrate knowledge of, and install, drainage in a sports turf area |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 5 |

| Purpose | People credited with this unit standard are able to demonstrate knowledge of the characteristics and causes of, and solutions for, poorly drained sports turf; describe drains, drainage, pipe drains and backfill materials for sports turf; and install a pipe drain for sports turf purposes. |
| --- | --- |

| Classification | Sports Turf > Sports Turf Construction and Drainage |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

None.

Outcomes and performance criteria

Outcome 1

Demonstrate knowledge of the characteristics and causes of, and solutions for, poorly drained sports turf.

Performance criteria

1.1	Identify the characteristics of poor drainage in terms of the symptoms.

Range	at least four of – black layer, ponding, gleying, mottling, species composition and health, thatch accumulation, smell, absence of earthworms, root distribution, wetness in profile, poor playability.

1.2	Describe poor drainage in terms of the causes.

Range	at least four of – water table height, rising water table caused by tides or ground water levels, soil type, impermeable subsoil, springs, seepage, runoff, soil and sand textural changes, overuse, traffic, pans, soil structural changes, tip sites, broken and blocked pipes, poor maintenance practices.

1.3	Determine a solution to poor drainage on a sports turf site from an analysis of the symptoms and causes.

Range	solutions may include – catchwater, interceptor, water table control, sub-surface profile reconstruction, biological, soil conditioning, physical treatments, outlet, surface shaping;

evidence of one is required.

Outcome 2

Describe drains, drainage, pipe drains, and backfill materials for sports turf.

Performance criteria

2.1	Describe drains in terms of the options available for sports turf areas.

Range	at least five of – pipe drains, mole drains, slit drains, strip drains, open drains, soak holes, V-drains, gradient, junctions, gravel bands.

2.2	Describe pipe drain materials in terms of their suitability for sports turf areas.

Range	at least three of – plastic, clay, concrete, perforated, corrugated, smooth, twin-walled.

2.3	Describe pipe drainage in terms of the features of design and installation.

Range	at least four of – herringbone, grid, parallel, random, fan, depth of excavation, junctions, gradient.

2.4	Describe optimum sports turf drainage backfill material characteristics in terms of their physical properties.

Range	at least three of – permeability, stability, size, availability, compatibility.

Outcome 3

Install a pipe drain for sports turf purposes.

Performance criteria

3.1	Select tools to suit trench specifications.

3.2	Lay out trench lines and excavate a clean trench to the specified width and depth for sports turf purposes in accordance with design specifications.

3.3	Excavate the trench to a positive outlet with a minimum fall in accordance with the requirements of the specified pipe drainage material and design specifications.

3.4	Position excavated material conveniently without interfering with drain-laying.

3.5	Lay the pipe on an even gradient in accordance with design specifications.

3.6	Install the specified backfill without contamination.

3.7	Restore the turf surface without sinkage.

| Planned review date | 31 December 2025 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 4 August 1995 | 31 December 2019 |
| Revision | 2 | 4 August 1995 | 31 December 2019 |
| Revision | 3 | 16 September 1996 | 31 December 2019 |
| Revision | 4 | 10 December 1997 | 31 December 2019 |
| Revision | 5 | 8 June 1999 | 31 December 2019 |
| Review | 6 | 15 December 2000 | 31 December 2019 |
| Revision | 7 | 12 January 2006 | 31 December 2019 |
| Review | 8 | 17 October 2008 | 31 December 2019 |
| Review | 9 | 25 January 2018 | 31 December 2023 |
| Review | 10 | 24 June 2021 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0052 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Primary Industry Training Organisation standards@primaryito.ac.nz if you wish to suggest changes to the content of this unit standard.