| Title | Test, locate and diagnose faults in electrical machine windings |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 2 |

| Purpose | This unit standard is intended for people who have responsibility for preparing machines for rewinding. People credited with this unit standard are able to: – test electrical machine windings in accordance with industry practice; and – locate and diagnose faults in electrical machine windings in accordance with industry practice. |
| --- | --- |

| Classification | Electrical Engineering > Electrical Machines |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	This unit standard has been developed for learning and assessment on-job or off-job in a simulated environment.

2	Definitions

Electrical machines – are defined as electric motors, generators, alternators, and transformers.

Industry practice – those practices that competent practitioners within the industry recognise as current industry best practice.

3	Assessment

The number and type of machines chosen are left to the discretion of the assessor, but must be sufficient to assess competence in all outcomes of the unit standard.

Outcomes and performance criteria

Outcome 1

Test electrical machine windings in accordance with industry practice.

Performance criteria

1.1	Carry out tests.

Range	tests – continuity of supply conductors, insulation resistance between windings and earth, resistance of windings; growler test on machines containing armatures.

1.2	Record results of tests.

Outcome 2

Locate and diagnose faults in electrical machine windings in accordance with industry

practice.

Range	faults – open circuit, short circuit, leakage to earth, differences in readings between windings;

at least one of each type of fault is required.

Performance criteria

2.1	Interpret test results and locate faults.

2.2	Diagnose the cause of fault from test results and visual inspection.

2.3	Tag machines and complete documentation.

| Planned review date | 31 December 2025 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 29 April 1994 | 31 December 2013 |
| Review | 2 | 23 April 1996 | 31 December 2013 |
| Review | 3 | 10 February 1999 | 31 December 2013 |
| Revision | 4 | 3 April 2001 | 31 December 2013 |
| Review | 5 | 26 May 2005 | 31 December 2013 |
| Review | 6 | 22 August 2008 | 31 December 2013 |
| Revision | 7 | 16 April 2010 | 31 December 2023 |
| Rollover and Revision | 8 | 15 March 2012 | 31 December 2023 |
| Revision | 9 | 15 January 2014 | 31 December 2023 |
| Review | 10 | 22 August 2019 | N/A |
| Review | 11 | 30 January 2025 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0003 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Waihanga Ara Rau Construction and Infrastructure Workforce Development Council qualifications@waihangaararau.nz if you wish to suggest changes to the content of this unit standard.