| Title | Make ready and print four or more colour process dry offset work |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 25 |

| Purpose | People credited with this unit standard are able to: make ready a multicolour dry offset press for four or more colour process work; print four or more colour process work on a multicolour dry offset press; and solve problems encountered during the print run. |
| --- | --- |

| Classification | Printing > Dry Offset |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

| Prerequisites | Unit 340, Demonstrate knowledge of safe working practices in the print industry, or demonstrate equivalent knowledge and skills. |
| --- | --- |

Guidance Information

1	All workplace practices must meet any applicable and recognised codes of practice, and documented workplace health, safety, and environmental procedures for personal, product, workplace health, safety, and environmental matters, and the obligations required under current law including the Health and Safety in Employment Act 1992, Resource Management Act 1991, and their subsequent amendments.

2	Workplace practices refer to the documented procedures for the machine and/or workplace.

Outcomes and performance criteria

Outcome 1

Make ready a multicolour dry offset press for four or more colour process work.

Performance criteria

1.1	Trapping and sequence order of inks are determined to ensure that the job requirements are met.

Range	slur, trapping, ink tack, surface of substrates, dot gain effects, areas of critical register, final colours required.

1.2	Make ready is carried out for four or more colour process work in accordance with workplace practices.

1.3	Quality of tonal values and trapping of secondary colours is checked against the job specifications, and faults found are rectified in accordance with workplace practices.

1.4	Make ready problems are solved in accordance with workplace practices.

Range	impression, position, preliminary colour settings.

Outcome 2

Print four or more colour process work on a multicolour dry offset press.

Performance criteria

2.1	Four or more colour process work is printed to meet the job specifications and it is ensured that production standards and quality are maintained throughout the run.

2.2	Colour consistency standards are maintained throughout the print run.

Range	dry back, dot gain, ink trapping.

2.3	Quality control procedures used to maintain required production standards are followed in accordance with workplace practices.

2.4	Bar code verifiers are used to ensure that printing of bar codes is maintained within required tolerances.

Outcome 3

Solve problems encountered during the print run.

Performance criteria

3.1	Problems identified while following workplace quality control procedures are recognised and solved.

Range	problems may include but are not limited to – dirty print, set-off, ghosting, gear marks, over and/or under impression, slur, ink contamination, bleeds, misregister, spots, show through, static, colour variation, image appearing washed out, product damage or contamination, ink problems, substrate problems, machine problems.

This unit standard is expiring.  Assessment against the standard must take place by the last date for assessment set out below.

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 26 February 1998 | 31 December 2025 |
| Revision | 2 | 20 April 2001 | 31 December 2025 |
| Revision | 3 | 19 November 2003 | 31 December 2025 |
| Review | 4 | 27 October 2006 | 31 December 2025 |
| Review | 5 | 30 March 2023 | 31 December 2027 |
| Rollover | 6 | 27 February 2025 | 31 December 2027 |

| Consent and Moderation Requirements (CMR) reference | 0005 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.