| Title | Demonstrate knowledge of timber machining |  |  |
| --- | --- | --- | --- |
| Level | 2 | Credits | 6 |

| Purpose | People credited with this unit are able to demonstrate knowledge of: principles of timber machining and safety practices; the operating principles of the timber machine; and factors affecting the finished quality of machined timber. |
| --- | --- |

| Classification | Wood Manufacturing - Generic Skills > Wood Manufacturing Foundation Skills |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Definitions

Accepted industry practice refers to approved codes of practice and standardised procedures accepted by the wider wood manufacturing industry as examples of best practice.

Timber machine refers to either a planer or a moulder.

Workplace procedures refer to documented policies and procedures set by the organisation carrying out the work, and to documented or other directions provided to staff, and applicable to the tasks being carried out.  They may include but are not limited to – standard operating procedures, site specific procedures, site safety procedures, equipment operating procedures, quality assurance procedures, product quality specifications, references, approved codes of practice, housekeeping standards, environmental considerations, on-site briefings, supervisor’s instructions, and procedures to comply with legislative and local body requirements relevant to the wood manufacturing sector.

2	Assessment information

All activities and evidence must meet workplace procedures and accepted industry practice.

Outcomes and performance criteria

Outcome 1

Demonstrate knowledge of timber machining and safety practices.

Performance criteria

  - Purpose of timber machining is explained.

Range	purpose – adding value to sawn timber product, meeting customer requirements.

1.2	Machined timber profiles are identified from given samples, for both structural and appearance grades of timber.

Range	structural – gauged, dressed four sides;

appearance – weatherboards (bevel back and rusticated), mouldings (scotia, architrave), tongue and groove profiles.

1.3	Safety practices required of personnel in the planer mill are identified.

Outcome 2

Demonstrate knowledge of the operating principles of the timber machine.

Performance criteria

2.1	Components of a timber machine are identified.

Range	components include but is not limited to – feed system, fences, pressures, chip breakers, extraction system, electrical controls, cutterheads and cutters.

2.2	Differences between timber machines are identified in terms of operating speeds and throughput size options.

2.3	Importance of correct orientation of feed stock to ensure best product and least downgrade.

Range	cup, crook, bow.

  - Purpose of docking is explained.

Range	purpose includes but is not limited to – preventing machine damage and product downgrade.

  - Basic machine maintenance requirements are explained.

Range	includes but is not limited to – housekeeping, lubrication, pulleys and drives, extraction system.

Outcome 3

Demonstrate knowledge of factors affecting the finished quality of machined timber.

Performance criteria

3.1	Effects on the finish quality of machine and timber variables are explained.

Range	machine variables – cutter pitch, feed speed;

timber variables – moisture content, grade, age.

  - Importance of balancing cutter equipment is described.

Range	personal danger, potential equipment damage, reduction in timber finish quality.

  - Factors determining feed speed are explained.

Range	end use applications, machine capability.

  - Machine defects are identified, and their cause explained.

Range	defects may include but are not limited to – sniping, burning, raised grain, chattering.

| Planned review date | 31 December 2024 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 10 February 1999 | 31 December 2014 |
| Revision | 2 | 12 December 2000 | 31 December 2014 |
| Review | 3 | 18 December 2006 | 31 December 2014 |
| Review | 4 | 18 April 2013 | N/A |
| Review | 5 | 28 May 2020 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0013 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Competenz qualifications@competenz.org.nz if you wish to suggest changes to the content of this unit standard.