| Title | Perform manual soldering and de-soldering procedures for electrotechnology work |  |  |
| --- | --- | --- | --- |
| Level | 2 | Credits | 2 |

| Purpose | This unit standard covers basic soldering for people working in the electrical and related trades. People credited with this unit standard are able to: – describe soldering procedures for electrical work – prepare and maintain soldering and de-soldering equipment – solder electrical components – perform soldering and de-soldering on printed circuit boards (PCBs). |
| --- | --- |

| Classification | Electrical Engineering > Core Electrical |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance information

1	This unit standard has been developed for learning and assessment off-job.

2	Reference

Health and Safety at Work Act 2015

and all subsequent amendments and replacements.

3	Definition

Industry practice – those practices that competent practitioners within the industry recognise as current industry best practice.

Outcomes and Performance criteria

Outcome 1

Describe soldering procedures for electrical work.

Performance criteria

1.1	Component preparation for soldering is described in accordance with industry practice.

Range	cleaned, tinned, devices heat-sinked.

1.2	Methods of preventing damage to electronic devices from excessive heat, electrostatic discharge, and mechanical stress are described in accordance with industry practice.

1.3	Methods of preventing short circuits and dry joints are described in terms of iron temperature, iron tip and solder selection, and solder joint shape.

1.4	Safety and workshop procedures to be followed while soldering are described in accordance with Health and Safety legislation.

Range	eye protection, ventilation, selection of clothing, work position.

Outcome 2

Prepare and maintain soldering and de-soldering equipment.

Performance criteria

2.1	Soldering and de-soldering equipment is chosen to suit the job in terms of the type of material and solder, bit size, heat requirement, and soldering location.

2.2	Soldering and de-soldering equipment is inspected to ensure user safety.

Range	electric cord set, vacuum hose where fitted, bits tight, handles undamaged and secure.

2.3	Soldering and de-soldering tips are kept tinned, shaped, and clean in accordance with industry practice.

Outcome 3

Solder electrical components.

Range	components may include but are not limited to – wiring lugs, connectors, connections.

evidence of ten solder joints is required.

Performance criteria

3.1	Solder, and flux where applicable, are chosen to match the job in terms of size and type of solder.

3.2	Materials are cleaned and secured in position ready for soldering task.

3.3	Materials are joined without damage to surrounding components and insulation, and solder is not overheated.

3.4	Soldered joints are completed in accordance with industry practice.

Range	joints are smooth; absence of – short circuits, dry joints, jagged edges, excess solder.

3.5	Safety and workshop procedures are followed in accordance with current Health and Safety legislation.

Outcome 4

Perform soldering and de-soldering on printed circuit boards (PCBs).

Range	solder at least ten components, de-solder at least ten components, bridge five PCB tracks.

Performance criteria

4.1	Anti-static precautions are followed in accordance with industry practice.

4.2	Resin-core solder is chosen to suit the job in terms of solder size.

4.3	Components are cleaned and secured in position in accordance with industry practice.

4.4	Components are soldered to PCBs without damage to surrounding components, tracks, and pads.

4.5	Joints are completed in accordance with industry practice.

Range	solder shape is filleted; component leads are trimmed; absence of short circuits, dry joints, jagged edges, excess solder.

4.6	Components are removed from PCBs and replaced without causing heat or stress damage to tracks and pads.

4.7	PCB bridges are installed in accordance with industry practice.

4.8	Safety and workshop procedures are followed while soldering in accordance with current Health and Safety legislation.

| Replacement information | This unit standard replaced unit standard 5923. |
| --- | --- |

| Planned review date | 31 December 2028 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 10 February 1999 | 31 December 2013 |
| Revision | 2 | 3 April 2001 | 31 December 2013 |
| Review | 3 | 26 May 2005 | 31 December 2013 |
| Revision | 4 | 26 January 2007 | 31 December 2025 |
| Rollover and Revision | 5 | 15 March 2012 | 31 December 2025 |
| Revision | 6 | 15 January 2014 | 31 December 2025 |
| Review | 7 | 28 March 2024 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0003 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Waihanga Ara Rau Construction and Infrastructure Workforce Development Council qualifications@WaihangaAraRau.nz if you wish to suggest changes to the content of this unit standard.