	NZQA unit standard
	17758 version 5

	
	Page 4 of 4



	Title
	Plan and form an extraction track in a forestry operation

	Level
	4
	Credits
	6


	Purpose
	This unit standard is intended for people operating earth moving machinery in a forestry operation.
People credited with this unit standard are able to plan the location of an extraction track for forestry operations, and form an extraction track for forestry operations.


	Classification
	Forestry > Machine Operations - Forestry


	Available grade
	Achieved


	Prerequisites
	One of:

Unit 6934, Operate a skidder in a forestry situation; or 
Unit 6935, Operate an excavator based tracked machine in a forestry operation; or 
Unit 6936, Operate a tracked machine in a forestry situation; or demonstrate equivalent knowledge and skills.


Guidance Information

1
Legislation relevant to this unit standard includes the: Health and Safety at Work (HSW) Act 2015, Resource Management Act 1991; the Resource Management (National Environmental Standards for Plantation Forestry) Amendment Regulations 2018; and the Heritage New Zealand Pouhere Taonga Act 2014, and any subsequent amendments.

2
References


New Zealand Forest Owners Association, Forest Practice Guides (2019), and any subsequent amendments, available from http://www.nzfoa.org.nz.
Approved Code of Practice (ACOP) for Safety and Health in Forestry Operations, December 2012, available from http://www.worksafe.govt.nz.
3
Definitions

Accepted industry practice refers to approved codes of practice and standardised procedures accepted by the wider forestry industry as examples of best practice.
Forestry operations refer to all activities forming a part of the work carried out in a commercial forest including: forest establishment, silviculture, harvesting, forest inventory, and forest and crew management.
Job prescription refers to any written instructions for the operation and may include maps, harvest plans or cut plans.
Worksite procedures refer to – documented procedures used by the organisation carrying out the work and applicable to the tasks being carried out.  They may include but are not limited to – standard operating procedures, site safety procedures, equipment operating procedures, quality assurance procedures, housekeeping standards, procedures to comply with legislative and local body requirements.
4
Assessment information

This unit standard must be assessed on-job.

Outcomes and performance criteria
Outcome 1

Plan the location of an extraction track for forestry operations.

Performance criteria
1.1
Extraction track location is planned to meet environmental requirements, job prescription, and operational requirements.

Range
regional authority requirements, forest owner requirements, riparian protection areas.

1.2
Cut-offs, soak holes, and water-tables are planned as required to minimise channelling of water runoff, in accordance with the job prescription and accepted industry practice.

1.3
Machinery suitable for track is formation is selected in accordance with accepted industry practice.
Range
type, size, power, stability, protective equipment.

1.4
The hazards and associated risks specific to track formation are identified and described and control methods are explained in accordance with accepted industry practice.

Range
includes but is not limited to – overhang, falling debris, subsidence, rolling stumps, rocks, unstable trees, soft ground conditions, undermining root plates, nearby forestry operations.
1.5
Factors to be considered when pushing over standing trees are explained in accordance with accepted industry practice.

Range
machine type and size, technique and machine placement, 
minimising stem damage, two tree length rule.
1.6
Obstructions on the site which require removal are explained in accordance with accepted industry practice.

Range
stumps, rock outcrops, fallen trees.

Outcome 2

Form an extraction track for forestry operations.

Performance criteria
2.1
Obstructions are removed with minimal stress and damage to machine in accordance with accepted industry practice.

Range
shock loading, hydraulic overload, blade or bucket damage.

2.2
Obstructions are managed to minimise hazards during future operations.

Range
may include but is not limited to – stability, placement.

2.3
Extraction tracks are formed to meet operational and environmental requirements.

Range
manageable gradients, stable formation, inward sloping, minimal visual impact, minimal soil disturbance, residual tree stability.

2.4
Hazards and associated risks are identified and controlled in accordance with accepted industry practice.

	Planned review date
	31 December 2028


Status information and last date for assessment for superseded versions

	Process
	Version
	Date
	Last Date for Assessment

	Registration
	1
	5 December 2000
	31 December 2020

	Review
	2
	22 May 2008
	31 December 2017

	Review
	3
	10 December 2015
	N/A

	Review
	4
	28 May 2020
	N/A

	Rollover
	5
	26 April 2024
	N/A


	Consent and Moderation Requirements (CMR) reference
	0173


This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Muka Tangata - People, Food and Fibre Workforce Development Council qualifications@mukatangata.nz if you wish to suggest changes to the content of this unit standard.

	Muka Tangata - People, Food and Fibre Workforce Development Council
SSB Code 6050
	SYMBOL 211 \f "Symbol" New Zealand Qualifications Authority 2024



