| Title | Demonstrate knowledge of basic electronic systems |  |  |
| --- | --- | --- | --- |
| Level | 2 | Credits | 5 |

| Purpose | People credited with this unit standard are able to demonstrate knowledge of: – resistive voltage dividers; – a resistor-capacitor circuit; – resistor-diode combinations; – transistor switches; and – d.c. voltage in a single transistor amplifier. |
| --- | --- |

| Classification | Electronic Engineering > Electronics Technology |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Suitable drawing symbols can be found in Robertson, M, Electronics for Young Entrepreneurs (2016), available from The Skills Organisation, www.skills.org.nz.

2	Definitions

Basic electronic systems refer to the following combinations of two or more electronic components:

- resistive voltage divider;

- capacitance charge-discharge circuit;

- resistor-diode combination;

- resistor-zener diode combination;

- single transistor used as switch or a basic amplifier;

d.c. – direct current;

LDR – light dependent resistor.

3	Assessment

Where needed, sketches and drawings may be used to aid explanations.

4	Range

a	All calculations and measurements must be expressed in Système International (SI) units and multipliers.

b	Use of resistor and capacitor colour coding charts is permitted during assessment.

Outcomes and performance criteria

Outcome 1

Demonstrate knowledge of resistive voltage dividers.

Performance criteria

1.1	Calculate the component values for a two-resistance voltage divider given input voltage and desired output voltage.

1.2	Calculate the output voltage of a two-resistance voltage divider from the input voltage and the resistance values.

1.3	Explain the behaviour of a voltage divider using an LDR and a fixed resistor with reference to changes in light level.

1.4	Take measurements to confirm the calculations carried out in performance criteria 1.1 and 1.2.

Outcome 2

Demonstrate knowledge of a resistor-capacitor circuit.

Performance criteria

2.1	Explain the charge and discharge characteristics of a capacitor through a resistor and confirm by measurement.

Range	characteristics – voltage vs time, and current vs time for charging and discharging.

2.2	Explain the concept of time constant in terms of the time to charge or discharge a capacitor.

2.3	Determine the time constant for a resistor-capacitor combination.

Outcome 3

Demonstrate knowledge of resistor-diode combinations.

Performance criteria

3.1	Explain the behaviour of resistor-diode combinations using circuit schematics and characteristic curves.

3.2	State the characteristics of a zener diode-resistor voltage regulator using circuit schematics and characteristic curves.

3.3	Take measurements to confirm the explanation and sketches.

Outcome 4

Demonstrate knowledge of transistor switches.

Performance criteria

4.1	Explain the operation of a transistor switch circuit in terms of its on and off conditions.

4.2	Take a measurement to confirm the explanation of performance criterion 4.1.

4.3	Describe the function of a transistor switch in a practical application.

Outcome 5

Demonstrate knowledge of d.c. voltage in a single transistor amplifier.

Range	voltage divider with a transistor emitter resistor.

Performance criteria

5.1	Explain the functions of all components.

5.2	Calculate collector, base, and emitter voltages in a given circuit.

5.3	Take measurements to confirm correct use of calculation procedures.

| Planned review date | 31 December 2025 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 30 April 2001 | 31 December 2012 |
| Revision | 2 | 12 March 2002 | 31 December 2012 |
| Revision | 3 | 17 March 2004 | 31 December 2012 |
| Review | 4 | 25 May 2007 | 31 December 2024 |
| Rollover and Revision | 5 | 15 March 2012 | 31 December 2024 |
| Revision | 6 | 15 January 2014 | 31 December 2024 |
| Rollover and Revision | 7 | 27 January 2015 | 31 December 2024 |
| Review | 8 | 24 June 2021 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0003 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact The Skills Organisation reviewcomments@skills.org.nz if you wish to suggest changes to the content of this unit standard.