| Title | Set a four sider to square dress wooden furniture components |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 4 |

| Purpose | People credited with this unit standard are able to: demonstrate and apply knowledge of preparing to set; and set a four sider, to square dress wooden furniture components. |
| --- | --- |

| Classification | Furniture > Furniture and Cabinet Making |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Assessment information

This unit standard must be assessed against on-job.

All activities and performance criteria must be carried out in accordance with worksite policies and procedures.

2	Personal protective equipment, appropriate to job requirements, must be selected and used in accordance with worksite policies and procedures and manufacturer’s instructions.

3	Definitions

Manufacturer’s instructions are instructions provided by manufacturers of substances, equipment, and machinery used in furniture making.  These instructions may include details on safe and correct handling, use and storage of substances and/or details on substance properties.  Examples are labels on substance containers, product data sheets, and operator’s manuals.

SOIs refer to safe operating instructions which are documented instructions attached to machines which detail requirements for safe operation of the particular machine.

Worksite policies and procedures refer to documented policies and to documented or other directions provided to staff.  These may include, but are not limited to, ways of managing health and safety, environmental considerations, quality, and production, and must conform to legislation.  Examples include standard operating procedures, company health and safety plans, on-site briefings, and supervisor’s instructions.

Outcomes and performance criteria

Outcome 1

Demonstrate and apply knowledge of preparing to set a four sider to square dress wooden furniture components.

Performance criteria

1.1	Job specifications are obtained and interpreted.

Range	material, component dimensions, number of components, timeframe.

1.2	Method for setting the machine is explained in accordance with manufacturer’s instructions.

Range	bottom cutter head, side cutter heads, top cutter head, feed rollers.

1.3	Method for adjusting the machine is explained in accordance with manufacturer’s instructions.

Range	width, thickness, feed speed.

1.4	Tooling and material are selected in accordance with job specifications.

1.5	Environmental and safety requirements are met.

Range	electrical power, lighting, ventilation, dust extraction.

Outcome 2

Set a four sider to square dress wooden furniture components.

Performance criteria

2.1	Machine is isolated from the power sources during set-up.

2.2	Straight knives are removed and replaced in the cutter head in accordance with manufacturer’s instructions and job specifications.

Range	equal projection, secure.

2.3	Cutter heads are secured onto the spindles in accordance with manufacturer’s instructions and job specifications.

2.4	Machine is set in accordance with job specifications, manufacturer’s instructions, and SOIs.

Range	feed rollers, feed speed, width, thickness, fences, safety guards.

2.5	Test piece is run and checked against job specifications.  Any required adjustments to machine settings are made.

Range	width of test piece, thickness of test piece, pitch of cutter marks.

2.6	Work area is made clean, clear, and safe.

| Replacement information | This unit standard and unit standard 18899 replaced unit standard 2233. |
| --- | --- |

| Planned review date | 31 December 2025 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 26 February 2002 | 31 December 2023 |
| Review | 2 | 19 June 2009 | N/A |
| Review | 3 | 28 January 2021 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0173 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Competenz qualifications@competenz.org.nz if you wish to suggest changes to the content of this unit standard.