| Title | Deglaze and reglaze leadlights |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 10 |

| Purpose | This unit standard is for experienced people in the glass processing industry working with decorative glass. People credited with this unit standard are able to: prepare for glazing leadlights; deglaze leadlights; reglaze leadlights; and complete job requirements. |
| --- | --- |

| Classification | Glass and Glazing > Glass Processing and Manufacturing |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Definition

Worksite requirements – refer to instructions to staff on policy and procedures which are documented in memo or manual format and are available in the workplace.  These requirements include – company specifications and procedures, work instructions, manufacturer’s specifications, product quality specifications, legislative requirements.

2	Legislation relevant to this unit standard includes – Health and Safety at Work Act 2015, Health and Safety in Employment Regulations 1995, Hazardous Substances and New Organisms Act 1996, Resource Management Act 1991.

Outcomes and performance criteria

Outcome 1

Prepare for glazing leadlights.

Performance criteria

1.1	Customer’s requirements are confirmed before work begins in accordance with the job specifications and worksite requirements.

1.2	All necessary equipment, tools and installation materials are checked and assembled prior to commencement of work in accordance with worksite requirements.

1.3	Work area is prepared to prevent damage to surrounding area and protect public safety in accordance with worksite requirements.

Range	surrounding area includes – furnishings, fixtures and fittings, exterior surfaces;

measures to protect public safety include – warning signs, barriers or cordons.

Outcome 2

Deglaze leadlights.

Performance criteria

2.1	Protective clothing and equipment necessary for the task is used in accordance with legislative and worksite requirements.

Range	protective clothing and equipment includes – overalls, full face mask, gloves, protective apron.

2.2	Original format and construction of the leadlight is recorded prior to removal in accordance with worksite requirements.

2.3	Old compound and retaining devices are removed without damage to periphery lead cames in accordance with worksite requirements.

2.4	All tacks are removed at lead joints in accordance with worksite requirements.

2.5	Leadlight is supported during removal in accordance with worksite requirements.

Outcome 3

Reglaze leadlights.

Performance criteria

3.1	Protective clothing and equipment necessary for the task is used in accordance with legislative and worksite requirements.

Range	protective clothing and equipment includes – overalls, full face mask, gloves, protective apron.

3.2	Frame is reprimed to cover all exposed timber and/or metal in accordance with worksite requirements.

3.3	Saddlebars are fitted to frame to suit leadlight size and assembly requirements in accordance with worksite requirements.

3.4	Frame is bedded with compound or rubber prior to positioning leadlight in accordance with worksite requirements.

3.5	Leadlight is centred and secured at lead came joint positions in accordance with worksite requirements.

3.6	Leadlight is fixed in position using fixings suitable for the glazing system in accordance with worksite requirements.

3.7	Face putty or rubber gaskets are applied to periphery lead cames to provide a continuous seal in accordance with worksite requirements.

Outcome 4

Complete job requirements.

Performance criteria

4.1	Work is completed in accordance with job specifications and customer's requirements.

4.2	Machinery, equipment and materials are cleaned and stored in accordance with manufacturer's and worksite requirements.

4.3	Post-processing clean-up ensures work area is left free of debris in accordance with worksite requirements.

4.4	Work is completed without injury to personnel and damage to product, equipment, and machinery in accordance with worksite requirements.

| Planned review date | 31 December 2022 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 11 June 1995 | 31 December 2013 |
| Revision | 2 | 5 February 1997 | 31 December 2013 |
| Revision | 3 | 20 December 1999 | 31 December 2013 |
| Review | 4 | 27 April 2004 | 31 December 2013 |
| Review | 5 | 19 June 2009 | 31 December 2017 |
| Review | 6 | 18 June 2015 | N/A |
| Rollover and Revision | 7 | 25 August 2022 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0073 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Hanga-Aro-Rau Manufacturing, Engineering and Logistics Workforce Development Council at qualifications@hangaarorau.nz if you wish to suggest changes to the content of this unit standard.