| Title | Defuel aircraft |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 4 |

| Purpose | People credited with this unit standard are able to: prepare to defuel aircraft; defuel aircraft; and complete finishing activities related to defuelling aircraft. |
| --- | --- |

| Classification | Aeronautical Engineering > Aircraft Mechanical Maintenance |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

- All tasks must be carried out in accordance with enterprise procedures.

- Definition

Enterprise procedures – procedures used by the organisation carrying out the work and applicable to the tasks being carried out.  Examples are – standard operating procedures, safety procedures, equipment operating procedures, codes of practice, quality management practices and standards, procedures to comply with legislative and local body requirements.

3	This unit standard may be assessed against the use of pressurised or gravity systems.

4	The scope of the system that this standard relates to is described in ATA iSpec 2200, chapter 12.

Outcomes and performance criteria

Outcome 1

Prepare to defuel aircraft.

Performance criteria

1.1	Task is determined by reviewing maintenance documentation.

1.2	Environmental conditions in which the task is to be carried out are checked for suitability.

Range	may include but is not limited to – precipitation, lightning, airborne dust and/or debris.

1.3	Resources are obtained and checked for serviceability or status.

Range	may include but is not limited to – publications, tools, safety equipment, support equipment, defuelling equipment.

1.4	Quantity or weight of fuel to be unloaded is calculated.

Range	may include but is not limited to – gallons, litres, pounds, kilograms, specific gravity.

1.5	Aircraft registration and system to be defuelled are matched with documentation.

1.6	Aircraft and systems are prepared for the application and removal of power and for system operation.

Range	may include but is not limited to – cockpit controls match component positions, clearances obtained, isolation tags fitted and/or removed, warning signs positioned, aircraft levelled, aircraft positioned, ignition sources eliminated, aircraft earthed.

1.7	Aircraft and ground equipment are positioned ready for defuelling.

Range	may include but is not limited to – pumps, ground power units, stands, fire extinguishers.

Outcome 2

Defuel aircraft.

Performance criteria

2.1	Fuel, containers, and equipment are matched with system to be defuelled.

2.2	Aircraft is defuelled.

Range	may include but is not limited to – fuel quality tested to determine if disposal is required, bonding attached, defuelling equipment connected, defuelling sequence and quantity monitored and controlled.

2.3	Initial action to be taken in an abnormal situation is demonstrated.

Range	may include but is not limited to – fire, spillage, personal contamination by fuel.

Outcome 3

Complete finishing activities related to defuelling aircraft.

Performance criteria

3.1	Aircraft, system, and work areas are left in a state that enables the next task to begin.

3.2	Resources are checked for serviceability and returned to service or storage.

3.3	Any defects and abnormal situations are reported and documented.

3.4	Documentation is completed.

| Replacement information | This unit standard and unit standard 20634 replaced unit standard 3910. |
| --- | --- |

| Planned review date | 31 December 2027 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 20 April 2006 | 31 December 2016 |
| Review | 2 | 18 June 2014 | 31 December 2021 |
| Review | 3 | 26 March 2020 | N/A |
| Rollover and Revision | 4 | 26 April 2024 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0028 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Ringa Hora Services Workforce Development Council qualifications@ringahora.nz if you wish to suggest changes to the content of this unit standard.