| Title | Refuel aircraft |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 4 |

| Purpose | People credited with this unit standard are able to: prepare to refuel aircraft; refuel aircraft; and complete activities related to refuelling. |
| --- | --- |

| Classification | Aeronautical Engineering > Aircraft Servicing |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

- All tasks must be carried out in accordance with enterprise procedures.

- Definition

Enterprise procedures – procedures used by the organisation carrying out the work and applicable to the tasks being carried out.  Examples are – standard operating procedures, safety procedures, equipment operating procedures, codes of practice, quality management practices and standards, procedures to comply with legislative and local body requirements.

3	This unit standard may be assessed against the use of pressurised or gravity systems.

4	Evidence for performance criterion 2.3 may be gathered under simulated conditions.

5	The scope of the system that this standard relates to is described in ATA iSpec 2200, chapter 12.

Outcomes and performance criteria

Outcome 1

Prepare to refuel aircraft.

Performance criteria

1.1	Task is determined by reviewing servicing documentation.

1.2	Environmental conditions in which the task is to be carried out are checked for suitability.

Range	may include but is not limited to – precipitation, lightning, airborne dust and/or debris.

1.3	Resources are obtained and checked for serviceability or status.

Range	may include but is not limited to – publications, tools, safety equipment, support equipment, refuelling equipment.

1.4	Quantity or weight of fuel to be loaded is calculated.

Range	may include but is not limited to – gallons, litres, pounds, kilograms, specific gravity.

1.5	Aircraft registration and system to be refuelled are matched with documentation.

1.6	Aircraft and systems are prepared for the application and removal of power and for system operation.

Range	may include but is not limited to – cockpit controls match component positions, clearances obtained, isolation tags fitted or removed, warning signs positioned, aircraft levelled, aircraft positioned, ignition sources eliminated, aircraft earthed.

1.7	Aircraft and ground equipment are positioned ready for refuelling.

Range	may include but is not limited to – pumps, ground power units, stands, fire extinguishers.

Outcome 2

Refuel aircraft.

Performance criteria

2.1	Fuel, containers, and equipment are matched with system to be refuelled.

2.2	Aircraft is refuelled.

Range	may include but is not limited to – fuel quality checked, bonding attached, refuelling equipment connected, distribution sequence and quantity monitored and controlled.

2.3	Initial action to be taken in an abnormal situation is demonstrated.

Range	may include but is not limited to – fire, spillage, personal contamination by fuel.

Outcome 3

Complete activities related to refuelling.

Performance criteria

3.1	Aircraft, system, and work areas are left in a state that enables the next task to begin.

3.2	Resources are checked for serviceability and returned to service or storage.

3.3	Any defects and abnormal situations are reported and documented.

3.4	Documentation is completed.

| Replacement information | This unit standard and unit standard 20633 replaced unit standard 3910. |
| --- | --- |

| Planned review date | 31 December 2027 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 20 April 2006 | 31 December 2016 |
| Review | 2 | 18 June 2014 | 31 December 2021 |
| Review | 3 | 26 March 2020 | N/A |
| Rollover and Revision | 4 | 26 April 2024 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0028 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Ringa Hora Services Workforce Development Council qualifications@ringahora.nz if you wish to suggest changes to the content of this unit standard.