| Title | Base out individual component proprietary scaffolding |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 6 |

| Purpose | People credited with this unit standard are able to: – position and stand standards for individual component proprietary scaffolding; – position and attach ledgers, transoms, and braces for individual component proprietary scaffolding; – fit and secure platform components for individual component proprietary scaffolding; and – check that the structure is compliant. |
| --- | --- |

| Classification | Lifting Equipment > Elementary Scaffolding |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	This unit standard has been developed for learning and assessment on-job or off-job in a simulated environment.  Where supervision is required by law, the supervisor must hold the appropriate Certificate of Competence for the scaffolding work undertaken.

2	All tasks are to be carried out in accordance with:

a	quality management systems;

b	designer requirements and manufacturer operating instructions; legislation, regulations, bylaws, Health and Safety at Work Act 2015, and Health and Safety in Employment Regulations 1995;

c	the most up to date version of the Good Practice Guidelines - Scaffolding in New Zealand (GPG), 2016 available from https://www.worksafe.govt.nz/topic-and-industry/working-at-height/scaffolding-in-new-zealand/;

and all subsequent amendments and replacements.

3	Definitions

Scaffolding is as defined in the GPG and in the Health and Safety in Employment Regulations 1995.

Scaffold plan is a key design document prepared by the candidate and used as a basis for the erection of a particular scaffold.

4	Training and assessment

This unit standard is intended to cover all work done on the ground in the erection of individual component proprietary scaffolding structures, for example prior to the point of leaving the ground by mounting the structure to work from the base lift.

Individual component scaffolding systems may include but are not limited to – kwikstage; rapid scaffold; cuplock, ring or all round scaffold.

Outcomes and performance criteria

Outcome 1

Position and stand standards for individual component proprietary scaffolding.

Range	at least consistent with the required off-set distance, width of the scaffold and the position of any existing standards.

Performance criteria

1.1	Lay out components for efficient erection.

1.2	Stand standards in correct sequence.

Outcome 2

Position and attach ledgers and transoms for individual component proprietary scaffolding.

Performance criteria

2.1	Position and attach ledgers and transoms in correct sequence.

2.2	Level and align structure.

Outcome 3

Position and attach braces for individual component proprietary scaffolding.

Performance criteria

3.1	Position and attach braces.

Range	may include but is not limited to – transverse bracing, longitudinal bracing, plan bracing.

3.2	Level and align structure.

Outcome 4

Fit and secure platform components for individual component proprietary scaffolding.

Performance criteria

4.1	Install platform components.

4.2	Secure and fix platform components.

Outcome 5

Check that the structure is compliant.

Performance criteria

5.1	Check the structure for compliance.

Range	in relation to the GPG inspection report, the scaffolding plan and intended use.

| Replacement information | This unit standard, unit standard 20857, unit standard 20858, unit standard 20859, unit standard 23223, unit standard 23224, unit standard 23225, unit standard 23226, and unit standard 23227 replaced unit standard 4201 and unit standard 4203. |
| --- | --- |

| Planned review date | 31 December 2026 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 23 March 2006 | 31 December 2016 |
| Revision | 2 | 25 July 2007 | 31 December 2016 |
| Rollover and Revision | 3 | 20 June 2008 | 31 December 2016 |
| Review | 4 | 16 July 2015 | 31 December 2019 |
| Review | 5 | 23 November 2017 | 31 December 2025 |
| Review | 6 | 24 February 2022 | N/A |
| Revision | 7 | 24 August 2023 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0003 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact the Waihanga Ara Rau Construction and Infrastructure Workforce Development Council qualifications@waihangaararau.nz if you wish to suggest changes to the content of this unit standard.