| Title | Process scrap metal using gas cutting equipment |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 5 |

| Purpose | This unit standard is for people working in the scrap metal recycling industry. People credited with this unit standard are able to: prepare for cutting operations; cut scrap metal products using gas cutting equipment; and demonstrate shutdown procedures and clean up after cutting scrap metal. |
| --- | --- |

| Classification | Resource Recovery > Scrap Metal Recycling |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

- All work practices must comply with:

Hazardous Substances and New Organisms Act 1996;

Health and Safety at Work Act 2015;

Resource Management Act 1991.

- Hazard controls, safety procedures, and personal protective equipment must be used in accordance with company procedures.

- Assessment against this unit standard requires evidence of cutting in a scrap metal yard and one other location.

- Definition

Company procedures mean the documented methods for performing work activities and include health and safety, environmental, and quality management requirements.  They may refer to manuals, codes of practice, or policy statements.

Outcomes and performance criteria

Outcome 1

Prepare for cutting operations.

Performance criteria

1.1	Work instructions are confirmed and scrap metal to be cut is inspected in accordance with company procedures.

Range	instructions may include but are not limited to – location, safety signage, barricade, finished size;

inspection may include but is not limited to – condition, shape, size, hazards.

1.2	Tools and equipment are selected to match job requirements and are checked for serviceability, and any corrective action required is taken in accordance with company procedures.

Range	check includes but is not limited to – personal protective equipment, fire extinguisher, cutting equipment.

1.3	Health and safety hazards are identified and controlled in accordance with legislation, site requirements, and company procedures.

Range	includes but is not limited to – emergency procedures.

1.4	Adverse effects on the environment are identified and controlled in accordance with legislation, site requirements, and company procedures.

1.5	Gas cutting equipment is set up and tested in accordance with company procedures.

Range	cylinders, regulators, purging, testing for leaks, selecting pressure, selecting cutting tip.

1.6	Scrap metal products are positioned ready for cutting in accordance with company procedures.

Outcome 2

Cut scrap metal products using gas cutting equipment.

Range	products – tank, plate, machinery, reinforcing steel, structural steel;

evidence is required of three different products;

evidence is required of vertical and horizontal cutting.

Performance criteria

2.1	Torch is lit safely in accordance with company procedures.

2.2	Flame is adjusted for cutting in accordance with company procedures.

2.3	Scrap metal is cut to size in accordance with work instructions.

2.4	Characteristics, uses, and limitations of gas cutting equipment are explained in terms of recycling scrap metal.

2.5	Scrap metal products are cut safely and economically in accordance with company procedures.

Outcome 3

Demonstrate shutdown procedures and clean up after cutting scrap metal.

Performance criteria

3.1	Torch is switched off and gas supply is shut off in accordance with company procedures.

3.2	Work area is cleared in accordance with site requirements and company procedures.

3.3	Work area is checked to ensure fire safety in accordance with site requirements and company procedures.

3.4	Tools and equipment are cleaned, checked, maintained, and stored in accordance with company procedures.

This unit standard is expiring.  Assessment against the standard must take place by the last date for assessment set out below.

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 26 January 2007 | 31 December 2012 |
| Revision | 2 | 20 May 2011 | 31 December 2025 |
| Rollover and Revision | 3 | 28 March 2019 | 31 December 2025 |
| Review | 4 | 27 March 2025 | 31 December 2025 |

| Consent and Moderation Requirements (CMR) reference | 0014 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.