| Title | Ground run turbo-jet or turbo-fan aircraft engines |  |  |
| --- | --- | --- | --- |
| Level | 5 | Credits | 10 |

| Purpose | People credited with this unit standard are able to: prepare to ground run turbo-jet or turbo-fan aircraft engines; carry out engine ground run; and complete activities related to the ground run task. |
| --- | --- |

| Classification | Aeronautical Engineering > Aircraft Servicing |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

- All tasks must be carried out in accordance with enterprise procedures.

- Definitions

Enterprise procedures – procedures used by the organisation carrying out the work and applicable to the tasks being carried out.  Examples are – standard operating procedures, safety procedures, equipment operating procedures, codes of practice, quality management practices and standards, procedures to comply with legislative and local body requirements.

FOD – foreign object debris which is anything that can find its way into an aircraft engine or flight control mechanisms that could possibly cause damage to aircraft, equipment, or people.

3	Approval to ground run specific aircraft and engine types is obtained from the enterprise responsible for maintaining the engine.

4	This unit standard covers generic procedures for ground running aircraft engines.

5	Turbo-jet or turbo-fan aircraft engines include single and/or multi engine, e.g. Soko Galeb, Citation, Boeing 757, Boeing 777.

6	The scope of the system that this standard relates to is described in the applicable chapters of ATA iSpec 2200.

Outcomes and performance criteria

Outcome 1

Prepare to ground run turbo-jet or turbo-fan aircraft engines.

Performance criteria

1.1	Ground run task is determined by reviewing maintenance documentation and enterprise procedures.

- 1.2	Resources are obtained and checked for serviceability or status.

Range	may include but is not limited to – publications, tools, equipment, safety equipment, materials.

- 1.3	Aircraft registration is matched with documentation.

- 1.4	Aircraft is prepared for engine start.

Range	may include but is not limited to – fuel load and location; blanks, locks, chocks, covers, screens; special equipment and safety equipment removed, fitted and positioned as necessary; engine start equipment and aircraft positioned in approved area, area checked for FOD before starting.

1.5	Environmental data is obtained.

Range	may include but is not limited to – barometric pressure, outside air temperature, wind speed and direction, humidity.

1.6	Ground run team is assembled and positioned.

Outcome 2

Carry out engine ground run.

Performance criteria

- 2.1	Engine start and ground run clearances are obtained.

Range	from ground crew and/or from control tower.

- 2.2	Engine or engines are started, run, and shut down to meet the determined task requirements or specifications.

- 2.3	Engine performance parameters are recorded.

2.4	Post ground run checks are completed.

Outcome 3

Complete activities related to the ground run task.

Performance criteria

3.1	Aircraft is prepared for next maintenance task or for operation.

- 3.2	Completion activities specific to the task and work area are carried out.

Range	may include but are not limited to – tool control, cleanliness, tidiness, return of publications, preparation for next activity.

- 3.3	Resources are checked for serviceability and returned to service or storage.

Range	may include but is not limited to – tools, equipment, safety equipment.

- 3.4	Ground run documentation is completed.

| Replacement information | This unit standard, unit standard 23165, and unit standard 23166, replaced unit standard 3905. |
| --- | --- |

| Planned review date | 31 December 2027 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 25 September 2006 | 31 December 2016 |
| Review | 2 | 18 June 2014 | 31 December 2021 |
| Review | 3 | 26 March 2020 | N/A |
| Rollover and Revision | 4 | 26 April 2024 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0028 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Ringa Hora Services Workforce Development Council qualifications@ringahora.nz if you wish to suggest changes to the content of this unit standard.