| Title | Erect and dismantle mobile scaffolding |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 4 |

| Purpose | People credited with this unit standard are able to: - check the stability of mobile scaffolding; – establish a safe area on the next working lift; - build the working lift; – check the structure is compliant and complete the GPG inspection report; – ready the lift above; and - dismantle mobile scaffolding. |
| --- | --- |

| Classification | Lifting Equipment > Elementary Scaffolding |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	This unit standard has been developed for learning and assessment on-job or off-job in a simulated environment.  Where supervision is required by law, the supervisor must hold the appropriate Certificate of Competence for the scaffolding work undertaken.

2	All tasks are to be carried out in accordance with:

a	quality management systems;

b	designer requirements and manufacturer operating instructions; legislation, regulations, bylaws, Health and Safety at Work Act 2015, and Health and Safety in Employment Regulations 1995;

c	the most up to date version of the Good Practice Guidelines - Scaffolding in New Zealand (GPG), 2016 available from https://www.worksafe.govt.nz/topic-and-industry/working-at-height/scaffolding-in-new-zealand/;

and all subsequent amendments and replacements.

3	Definition

Scaffolding is as defined in the GPG and in the Health and Safety in Employment Regulations 1995.

4	Training and assessment

Competency for this unit standard must be demonstrated using two different types of scaffolding.  Types of scaffolding include but are not limited to – aluminium, proprietary and tube and coupler scaffolds.

This unit standard focuses on erection and dismantling skills after the completion of the base lift, the erection of which is dealt with in previous unit standards.

5	Recommended skills and knowledge

Unit 20858, Base out tube and coupler scaffolding; Unit 20859, Base out proprietary frame scaffolding; Unit 20860, Base out individual component proprietary scaffolding; Unit 20861, Demonstrate, at height, safety procedures for the erection and dismantling of scaffolding; and Unit 23228, Complete a scaffolding inspection compliance report; or demonstrate equivalent knowledge and skills.

Outcomes and performance criteria

Outcome 1

Check the stability of mobile scaffolding.

Performance criteria

1.1	Calculate the safety factor with allowance made for adverse conditions.

1.2	Select and install castors.

1.3	Choose rakers or outriggers.

Outcome 2

Establish a safe area on the next working lift.

Performance criteria

2.1	Access the lift above internally, through the planking and via a ladder.

2.2	Attach an initial guardrail.

2.3	Close the stop end.

Outcome 3

Build the working lift.

Range	using correct sequence consistent with the scaffold plan; external components fitted first.

Performance criteria

3.1	Fit joiners to standards as appropriate to the type of scaffolding being used.

3.2	Fit standards, individual components or frames.

Range	standards and/or components are checked for vertical and are at heights consistent with the scaffold plan, pre-engineered components are fitted.

3.3	Walk out guardrails in sequence.

3.4	Fit mid rails, ledgers and transoms in sequence as appropriate to the type of scaffolding being used.

3.5	Lash or fit planks and kickboards on the working lift.

3.6	Brace the structure.

Range	may include – transverse, longitudinal, parallel braces; outriggers; rakers.

Outcome 4

Check the structure is compliant and complete the GPG inspection report.

Performance criteria

4.1	Check the structure for compliance with the GPG inspection report and the scaffold plan.

4.2	Complete the GPG inspection report.

4.3	Make changes where structure is not compliant to ensure compliance and amend the GPG inspection report accordingly.

Outcome 5

Ready the lift above.

Performance criteria

5.1	Check transoms for spacing and for square on vertical and horizontal.

5.2	Place planks on lift above.

5.3	Build internal access to the lift above.

Outcome 6

Dismantle mobile scaffolding.

Performance criteria

6.1	Dismantle the deck in sequence.

Range	all components removed progressively beginning with kickboards and mid rails from the top working lift.

6.2	Descend the dismantled deck to the lift below.

| Replacement information | This unit standard, unit standard 20857, unit standard 20858, unit standard 20859, unit standard 20860, unit standard 23223, unit standard 23224, unit standard 23226 and unit standard 23227 replaced unit standard 4201 and unit standard 4203. |
| --- | --- |

| Planned review date | 31 December 2026 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 25 July 2007 | 31 December 2016 |
| Review | 2 | 16 July 2015 | 31 December 2019 |
| Review | 3 | 23 November 2017 | 31 December 2025 |
| Review | 4 | 24 February 2022 | N/A |
| Revision | 5 | 24 August 2023 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0003 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Waihanga Ara Rau Construction and Infrastructure Workforce Development Council qualifications@waihangaararau.nz if you wish to suggest changes to the content of this unit standard.