| Title | Produce metal castings using gravity die casting machine |  |  |
| --- | --- | --- | --- |
| Level | 2 | Credits | 5 |

| Purpose | People credited with this unit standard are able to: prepare gravity die casting equipment; produce castings using gravity die casting machine; and clean the die body in the machine. This unit standard applies to gravity casting into a permanent die. |
| --- | --- |

| Classification | Mechanical Engineering > Metal Forming |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

- Unit 2373, Work safely with molten metals is recommended for entry into this unit standard.

- References and legislations

Health and Safety at Work Act 2015.

Hazardous Substances and New Organisms Act 1996.

Resource Management Act 1991.

WorkSafe New Zealand.  Health and Safety Guidelines on the Management of Hazards in the Metal Casting Industry.  Available at: http://www.business.govt.nz/worksafe/information-guidance/all-guidance-items/metal-casting-industry-health-and-safety-guidelines-for-the-management-of-hazards-in/metal-casting-hs-guidelines.pdf.

3	Definitions

Accepted industry practice refers to approved codes of practice and standardised procedures accepted by the wider mechanical engineering industry sectors as examples of best practice.

Workplace procedures refer to procedures used by the organisation carrying out the work and applicable to the tasks being carried out.  They may include but are not limited to – standard operating procedures, safety procedures, equipment operating procedures, codes of practice, quality management practices and standards, procedures to comply with legislative and local body requirements.

4	Assessment Information

a	All activities must comply with applicable workplace procedures and must be consistent with accepted industry practice.

b	The material used in gravity die casting is non-ferrous metal.

Outcomes and performance criteria

Outcome 1

Prepare gravity die casting equipment.

Performance criteria

1.1	Die is inspected to ensure that it matches with the job to be cast.

1.2	Die coat is mixed.

1.3	Die temperature is maintained at level scheduled on job sheet.

1.4	Personal Protective Equipment (PPE) is used.

1.5	Die coat is applied in sequence and in a safe manner.

1.6	Die is securely located on machine and closed.

Outcome 2

Produce castings using gravity die casting machine.

Performance criteria

2.1	Molten metal is transferred into die in machine.

2.2	Conditions that contribute to inferior and/or reject castings are monitored.

2.3	Solidification time meets job requirements.

2.4	Castings are removed from die and stored in a manner that minimises damage.

2.5	Die condition is monitored and coating reapplied as required.

Outcome 3

Clean the die body in the machine.

Performance criteria

3.1	PPE is used.

3.2	Body of die is cleaned using abrasive blaster.

3.3	Work area is cleaned of coating and abrasive blasting residue.

| Planned review date | 31 December 2021 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 25 October 1994 | 31 December 2012 |
| Revision | 2 | 14 April 1997 | 31 December 2012 |
| Revision | 3 | 5 January 1999 | 31 December 2012 |
| Review | 4 | 19 October 1999 | 31 December 2012 |
| Review | 5 | 19 May 2006 | 31 December 2016 |
| Rollover and Revision | 6 | 17 November 2011 | 31 December 2022 |
| Review | 7 | 15 September 2016 | N/A |
| Revision | 8 | 23 November 2017 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0013 |
| --- | --- |

Comments on this unit standard

Please contact Competenz qualifications@competenz.org.nz if you wish to suggest changes to the content of this unit standard.