| Title | Fill fire service breathing apparatus cylinders |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 3 |

| Purpose | People credited with this unit standard are able to: - demonstrate knowledge of the types of fire service breathing apparatus (BA) cylinders; - demonstrate knowledge of a fire service BA cylinder filling station; and - use a BA cylinder filling station. |
| --- | --- |

| Classification | Fire and Rescue Services > Fire and Rescue Services - Generic Fire Fighting |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Compliance with the fire and rescue service provider’s Health and Safety policy and procedures is mandatory.

2	Definition

Fire and emergency agency’s requirements refer to policies, procedures and supporting documentation on safety and operations set down by each fire and emergency agency employer or host organisation.

3	Assessment against outcome 3 must be in accordance with the fire and emergency agency’s requirements.

Outcomes and performance criteria

Outcome 1

Demonstrate knowledge of the types of fire service breathing apparatus (BA) cylinders.

Range	steel, alloy, composite.

Performance criteria

1.1	Cylinders used by the fire and rescue service provider are identified.

1.2	Cylinder marking information is located on each of the cylinders and described in terms of cylinder identification number, test station mark, working pressure, and test date.

1.3	The recommended frequency of testing and working pressure of each type of BA cylinder is given in accordance with manufacturer’s specifications.

Outcome 2

Demonstrate knowledge of a fire service BA cylinder filling station.

Performance criteria

2.1	The components of the BA cylinder filling station are identified and explained in terms of function and operation.

Range	may include but is not limited to – filtration unit, cooler, compressor, storage cylinders, filling panel, water bath.

2.2	The safety requirements of the BA cylinder filling station are explained.

Range	personal protective equipment, start-up procedures, emergency shut-down procedures, access to filling stations.

2.3	The principles of the cascade system are explained.

Outcome 3

Use a BA cylinder filling station.

Performance criteria

3.1	Pre-filling checks are performed.

Range	pre-filling documentation, filling station instructions, air purity certificate, cylinder checks.

3.2	Fire service BA cylinders are filled.

Range	 a minimum of two cylinders.

3.3	BA cylinder recoding documentation is completed.

| Planned review date | 31 December 2023 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 20 November 2009 | N/A |
| Revision and Rollover | 2 | 30 September 2021 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0039 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact The Skills Organisation reviewcomments@skills.org.nz if you wish to suggest changes to the content of this unit standard.