| Title | Maintain pressure measurement devices |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 10 |

| Purpose | People credited with this unit standard are able to: – reconfigure or calibrate pressure measurement devices; and – service and test pressure measurement devices and systems. |
| --- | --- |

| Classification | Industrial Measurement and Control > Industrial Measurement and Control - Maintenance |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	This unit standard has been developed for learning and assessment in a workplace environment.

2	References

#### ANSI/ISA-51.1-1979 (R1993) Process Instrumentation Terminology;

#### Electricity Act 1992;

#### Electricity (Safety) Regulations 2010;

#### Health and Safety at Work Act 2015 and associated regulations;

ISSN 0114-0663, New Zealand Electrical Codes of Practice, available from Worksafe, https://worksafe.govt.nz/;

and their subsequent amendments and replacements.

3	Definitions

Industry requirements – includes all asset owner requirements; manufacturers’ specifications; enterprise requirements which cover the documented workplace policies, procedures, specifications, and business requirements; and quality management requirements relevant to the workplace in which the assessment is carried out.

Service – planned activity during normal operation that involves inspection, cleaning, testing, adjusting, or making minor repairs to a piece of equipment to ensure that it works properly.

4	Recommended skills and knowledge: Unit 2630, Demonstrate knowledge of pressure measurement systems used in industry; Unit 28082, Demonstrate knowledge of transducers and strain gauges and their applications in industrial measurement; Unit 28078, Demonstrate knowledge of industrial measurement processes, standards, and calibration.

Outcomes and performance criteria

Outcome 1

Reconfigure or calibrate pressure measurement devices.

Range	one differential pressure transmitter, one pressure gauge, one pressure switch.

Performance criteria

1.1	Identify type and cause of typical errors.

Range	may include but is not limited to – zero, span, component wear.

1.2	Select test equipment according to accuracy and range of devices.

Range	may include but is not limited to – deadweight tester, manometer, air test equipment, dedicated test equipment.

1.3	Explain and follow safe work procedures.

Range	may include but is not limited to – process isolation, loop isolation.

1.4	Calibrate or reconfigure devices, as specified by the data sheet.

Range	devices may include but is not limited to – transmitter, gauges, pressure switches;

calibration may include but not limited to – zero, span, linearity, head correction.

1.5	Document test results in accordance with industry requirements.

Outcome 2

Service and test pressure measurement devices and systems.

Range	one differential pressure transmitter, one pressure gauge, one pressure switch.

Performance criteria

2.1	Locate, interpret, and apply technical information for servicing equipment.

2.2	Explain and follow safe work procedures.

Range	may include but is not limited to – isolation, filled systems, chemical pressure.

2.3	Service and test pressure measurement devices to ensure correct and continued operation.

Range	may include but is not limited to – tools, materials, parts, techniques (including tests), specifications.

2.4	Produce reports and documentation in accordance with industry requirements.

| Replacement information | This unit standard was replaced by skill standard 40238. |
| --- | --- |

This unit standard is expiring.  Assessment against the standard must take place by the last date for assessment set out below.

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 31 October 1995 | 31 December 2013 |
| Revision | 2 | 30 October 1997 | 31 December 2013 |
| Revision | 3 | 3 April 2001 | 31 December 2013 |
| Review | 4 | 22 June 2001 | 31 December 2013 |
| Review | 5 | 19 May 2008 | 31 December 2019 |
| Review | 6 | 21 November 2013 | 31 December 2027 |
| Rollover and Revision | 7 | 28 June 2018 | 31 December 2027 |
| Review | 8 | 30 January 2025 | 31 December 2027 |

| Consent and Moderation Requirements (CMR) reference | 0003 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.