| Title | Maintain and calibrate density monitoring equipment |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 4 |

| Purpose | People credited with this unit standard are able to: – calibrate density measuring devices; and – maintain density measuring devices. |
| --- | --- |

| Classification | Industrial Measurement and Control > Industrial Measurement and Control - Maintenance |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	This unit standard has been developed for learning and assessment in a workplace environment.

2	References

ANSI/ISA-51.1-1979 (R1993) Process Instrumentation Terminology;

Electricity Act 1992;

Electricity (Safety) Regulations 2010;

Health and Safety at Work Act 2015 and associated regulations;

ISSN 0114-0663, New Zealand Electrical Codes of Practice, available from Worksafe, https://worksafe.govt.nz/;

CSP19 – ISSN 0110-9316, Code of Safe Practice for the use of industrial gauges containing sealed radioactive sources 2005 (Ministry of Health, New Zealand);

and all subsequent amendments and replacements.

3	Definitions

Density measurement – can include nuclear thickness measuring equipment.

Industry requirements – includes all asset owner requirements, manufacturers’ specifications; and enterprise requirements which cover the documented workplace policies, procedures, specifications, business requirements; and quality management requirements relevant to the workplace in which the assessment is carried out.

Maintain – planned activity during normal operation, to conserve or keep an item or piece of equipment in a state of repair and to ensure that this is done in a sustainable way.

4	Recommended skills and knowledge: Unit 28080, Demonstrate knowledge of liquid analytical measurement systems.

Outcomes and performance criteria

Outcome 1

Calibrate density measuring devices.

Performance criteria

1.1	Explain and follow safe work procedures.

Range	may include but is not limited to – isolation, process fluid, pressure.

1.2	Select test equipment according to required accuracy and range of devices.

Range	may include but is not limited to – pressure calibration equipment, nuclear radiation sources.

1.3	Identify and explain types and causes of typical errors.

Range	may include but is not limited to – test equipment accuracy, ambient temperature.

1.4	Calibrate devices to specification.

Range	may include but is not limited to – zero, span, suppression, elevation.

1.5	Document test results in accordance with industry requirements.

Outcome 2

Maintain density measuring devices.

Performance criteria

2.1	Locate, interpret, and apply technical information for the device.

2.2	Explain and follow safe work procedures are explained and followed.

Range	may include but is not limited to – isolation, process fluid, pressure.

2.3	Maintain device to ensure continued operation.

Range	may include but is not limited to – tools, materials, parts, techniques, specifications.

2.4	Reinstate device to operational status in accordance with industry requirements.

2.5	Produce reports and documentation in accordance with industry requirements.

This unit standard is expiring.  Assessment against the standard must take place by the last date for assessment set out below.

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 31 October 1995 | 31 December 2013 |
| Revision | 2 | 30 October 1997 | 31 December 2013 |
| Revision | 3 | 3 April 2001 | 31 December 2013 |
| Review | 4 | 22 June 2001 | 31 December 2013 |
| Review | 5 | 19 May 2008 | 31 December 2019 |
| Review | 6 | 21 November 2013 | 31 December 2027 |
| Rollover and Revision | 7 | 28 June 2018 | 31 December 2027 |
| Review | 8 | 30 January 2025 | 31 December 2027 |

| Consent and Moderation Requirements (CMR) reference | 0003 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.