| Title | Calibrate and maintain test equipment in a civil engineering laboratory |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 8 |

| Purpose | People credited with this unit standard are able to, in a civil engineering laboratory: describe the procedures to operate test equipment; prepare test equipment for calibration; calibrate test equipment; report and document calibration results; and maintain test equipment. |
| --- | --- |

| Classification | Infrastructure Civil Engineering > Infrastructure Laboratory |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

- Evidence presented for assessment against this unit standard must be consistent with safe working practices and be in accordance with applicable organisational and legislative requirements.

- Applicable legislation, regulations, rules, standards and codes include but are not limited to the: Health and Safety at Work Act 2015, Hazardous Substances and New Organisms Act 1996, and their associated regulations and subsequent amendments;

ISO/IEC 17025:2018 General requirements for the competence of testing and calibration laboratories, available from https://www.iso.org/store.html;

NZS 4402:1986 Methods of testing soils for civil engineering purposes - Preliminary and general, available from https://www.standards.govt.nz/;

NZS 4407:2015 Methods of sampling and testing road aggregates, available from https://standards.govt.nz.

3	Evidence is required for test equipment in three categories from the following – equipment to measure mass, temperature, length, volume, pressure, force; and temperature-controlled cabinets, used in a civil engineering laboratory.

4	Definitions

Organisational requirements refer to instructions to staff on policy and procedures which are formally documented or generally accepted at the worksite.  This may include legislation; industry standards and methods; national and international standards and methods; standard operating procedures; specifications; manuals; and manufacturer’s information.

Reference standards refer to standardised objects or substances which are used as a measurement base for similar objects or substances.

Outcomes and performance criteria

- Outcome 1

Describe the procedures to operate test equipment in a civil engineering laboratory.

Performance criteria

1.1	The calibration procedure is described.

1.2	The maintenance procedure is described.

1.3	The quality control checks are identified, and the type and cause of typical errors are described.

- Outcome 2

- Prepare test equipment for calibration in a civil engineering laboratory.

- Performance criteria

2.1	The calibration procedure is selected.

2.2	Hazards are identified and personal protective equipment, safety equipment and procedures are used.

2.3	All measuring equipment is confirmed to meet the laboratory’s specification requirements, and the calibration procedure is fully complied with.

2.4	Reference standards and measuring equipment are assembled, and test equipment is set up and adjusted as necessary.

2.5	Potential sources of measurement error are identified and minimised.

- Outcome 3

- Calibrate test equipment in a civil engineering laboratory.

- Performance criteria

3.1	Individual tests are performed within acceptable variance to ensure repeatability of measurement.

3.2	Readings are confirmed as a result of a valid measurement and data is recorded.

3.3	Resulting test data is analysed.

- Range	includes but is not limited to – uncertainty of measurement, trends, inconsistencies, accuracy, precision, validity.

- Outcome 4

Report and document calibration results in a civil engineering laboratory.

Performance criteria

4.1	Results of each calibration are recorded.

4.2	Calibration labels, equipment stickers, quality control tags and seals are attached where required.

4.3	Compliance or non-compliance with requirements of test specifications is reported and documented, and the next course of action is discussed with supervisor, manager or colleagues.

- Outcome 5

- Maintain test equipment in a civil engineering laboratory.

- Performance criteria

5.1	Maintenance procedures and appropriate records are identified.

5.2	Equipment, facilities, reference standards and stocks of consumables are maintained.

5.3	Maintenance is planned and evaluated.

5.4	Equipment requiring maintenance is identified, documented and reported.

| Planned review date | 31 December 2025 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 21 January 2011 | 31 December 2021 |
| Review | 2 | 23 January 2020 | N/A |
| Rollover and Revision | 3 | 24 October 2024 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0101 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Waihanga Ara Rau Construction and Infrastructure Workforce Development Council at qualifications@waihangaararau.nz if you wish to suggest changes to the content of this unit standard.