| Title | Produce components by CNC wire cut electro-discharge machine operations |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 15 |

| Purpose | This unit standard covers production of components using a Computer Numerical Control (CNC) wire cut electro-discharge machine. Preparation of the machine program is included. People credited with this unit standard are able to prepare program for operation, prepare machine for cutting, cut components, and measure components. |
| --- | --- |

| Classification | Mechanical Engineering > Engineering Machining and Toolmaking |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	References

Health and Safety at Work Act 2015.

Best Practice Guidelines, Safe Use of Machinery, Worksafe, 2014.

2	Definitions

CNC – Computer Numerical Control.

Specifications – detail that defines an object being made; commonly communicated by annotated and dimensioned drawings; by written description, or by other communication media.  External references may also be used to specify objects such as tables or industry standards.

Workplace procedures – procedures used by the organisation carrying out the work and applicable to the tasks being carried out.  Examples are – standard operating procedures, safety procedures, equipment operating procedures, codes of practice, quality management practices and standards, procedures to comply with legislative and local body requirements.

Outcomes and performance criteria

Outcome 1

Prepare program for operation.

Performance criteria

1.1	Drawings and documentation are interpreted to establish job requirements and component geometry.

1.2	Wire path coordinates are defined to meet specifications.

1.3	Program elements and machine code functions are defined to meet specifications.

1.4	Power settings, feed rate, and wire speed are selected to produce optimum metal cutting rates with the material used, and having regard to machine parameters.

Range	parameters – surface finish, metal removal rate, workpiece material, electrode material, electrode wear, selection of di-electric, flushing rate and method, spark gap.

1.5	Program is prepared in accordance with workplace procedures.

1.6	Operation instructions are prepared in accordance with workplace procedures.

1.7	Program is verified as complying with specifications.

Range	simulator or on machine tool.

Outcome 2

Prepare machine for cutting.

Performance criteria

2.1	Accessories and work-holding fixtures are installed to securely hold the work.

2.2	Machine is aligned to specified datum points in accordance with workplace procedures.

2.3	Program is selected and loaded according to workplace procedures.

Outcome 3

Cut components.

Performance criteria

3.1	Components are cut to specifications without damage to the machine.

3.2	Machine is cleaned and waste material disposed of in accordance with workplace procedures.

Outcome 4

Measure components.

Performance criteria

4.1	Components are measured using instruments matching the required accuracy to confirm that specifications have been achieved.

4.2	Measurements are recorded in accordance with workplace procedures.

| Planned review date | 31 December 2022 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 17 January 1995 | 31 December 2012 |
| Revision | 2 | 14 April 1997 | 31 December 2012 |
| Revision | 3 | 13 November 1997 | 31 December 2012 |
| Revision | 4 | 5 January 1999 | 31 December 2012 |
| Revision | 5 | 25 September 2001 | 31 December 2012 |
| Review | 6 | 20 June 2006 | 31 December 2022 |
| Rollover and Revision | 7 | 17 November 2011 | 31 December 2022 |
| Review | 8 | 17 August 2017 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0013 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Competenz qualifications@competenz.org.nz if you wish to suggest changes to the content of this unit standard.