| Title | Produce components by performing engineering grinding operations |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 15 |

| Purpose | This unit standard is intended for workplace assessment and covers surface and cylindrical grinding operations to produce components to specified tolerances and finish. It may involve a centreless machine. People credited with this unit standard are able to prepare for, and perform, grinding operations, and verify grinding accuracy. |
| --- | --- |

| Classification | Mechanical Engineering > Engineering Machining and Toolmaking |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

| Prerequisites | Unit 29650, Demonstrate knowledge of the safe use of powered equipment in a mechanical engineering or fabrication workshop, or demonstrate equivalent knowledge and skills. |
| --- | --- |

Guidance Information

1	References

Health and Safety at Work Act 2015.

Accident Compensation Corporation and Department of Labour.  Metal Industry Guidelines for Safe Work.  (Wellington: ACC, 2007).  Available from http://www.acc.co.nz.

Culley R. (2010) Fitting and Machining. Melbourne, Australia, RMIT Publishing. ISBN 9781921426780.

2	Definitions

Specifications – detail that defines an object being made; commonly communicated by annotated and dimensioned drawings; by written description, or by other communication media.  External references may also be used to specify objects such as tables or industry standards.

Workplace procedures – procedures used by the organisation carrying out the work and applicable to the tasks being carried out.  Examples are – standard operating procedures, safety procedures, equipment operating procedures, codes of practice, quality management practices and standards, procedures to comply with legislative and local body requirements.

Outcomes and performance criteria

Outcome 1

Prepare for grinding operations.

Range	three examples of surface grinding and three of internal or external cylindrical grinding.

Performance criteria

1.1	Machine condition and capability are checked in accordance with job requirements and workplace procedures.

1.2	Specifications are interpreted to establish job requirements.

1.3	Sequence of operations is planned to achieve specifications efficiently, in accordance with workplace procedures.

1.4	Components are marked out in accordance with specifications.

1.5	Accessories and work-holding fixtures are installed to securely hold the work.

1.6	Grinding wheels are selected, dressed, balanced if necessary, and mounted securely in accordance with workplace procedures.

1.7	Grinding speeds and feeds are selected for efficient machining of the material used.

Outcome 2

Perform grinding operations.

Range	three examples of surface grinding and three of internal or external cylindrical grinding.

Performance criteria

2.1	Workplace safety procedures are followed.

Range	examples are – use of personal protective equipment, checking of equipment for faults, use of extraction equipment, elimination of risk of fire or explosion.

2.2	Damage to work and equipment is prevented in accordance with workplace procedures.

2.3	Machine coolant type and flow are adjusted as required by machine and grinding operations.

2.4	Components are machined and finished to tolerances, and demagnetised where appropriate, in accordance with workplace procedures.

2.5	Machine is cleaned and waste material disposed of in accordance with workplace procedures.

Outcome 3

Verify grinding accuracy.

Range	components ground for outcome 2.

Performance criteria

3.1	Components are measured using instruments matching the required accuracy to confirm that specifications have been achieved.

3.2	Measurements are recorded in accordance with workplace procedures.

| Planned review date | 31 December 2022 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 17 January 1995 | 31 December 2012 |
| Revision | 2 | 14 April 1997 | 31 December 2012 |
| Revision | 3 | 13 November 1997 | 31 December 2012 |
| Revision | 4 | 5 January 1999 | 31 December 2012 |
| Revision | 5 | 25 September 2001 | 31 December 2012 |
| Review | 6 | 20 June 2006 | 31 December 2022 |
| Rollover and Revision | 7 | 17 November 2011 | 31 December 2022 |
| Review | 8 | 20 July 2017 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0013 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Competenz qualifications@competenz.org.nz if you wish to suggest changes to the content of this unit standard.