| Title | Sew dissimilar fabrics together using an industrial overlock sewing machine |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 6 |

| Purpose | People credited with this unit standard are able to set up and adjust an industrial overlock sewing machine, and sew dissimilar fabrics together. |
| --- | --- |

| Classification | Clothing Manufacture > Apparel Cutting and Sewing |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Range

Thick or rigid – denim, 9 gauge knitted.

Thin or stretch	– lycra, nylon, lace, gauze single jersey.

Slippery	 – lycra, nylon.

Open –	net, lace.

2	Performance of the elements must comply with the Health and Safety in Employment Act 1992.

3	Definitions

Workplace procedures refer to the verbal or documented procedures for performing work activities and include health and safety, operational, environmental, and quality management requirements.  They may refer to manuals, manufacturer's specifications, codes of practice, or policy statements.

Workplace performance criteria refer to the expected standard and speed of performance as set by the quality control management in the candidate’s workplace.  These criteria are determined by the construction of the garment, the fabric being used and the quality standards that apply in the candidate's workplace.  These expectations must be explained and made available to the candidate prior to assessment.

Product, garment or style specifications are all terms for the same document.  The terminology varies between workplaces but they all refer to the same documentation which accompanies each manufacturing order.  This documentation sets out the material to be used for the product, the size range of the order, the product assembly sequence, product finishing procedures, and the component specifications.

Outcomes and performance criteria

Outcome 1

Set up and adjust an industrial overlock sewing machine.

Performance criteria

1.1	Needles and threads are compatible with the characteristics and end use of the materials to be sewn.

1.2	Machine is adjusted to accommodate fabric requirements and product specifications.

Range	tension, foot pressure, adjust feed dogs, stitch density.

1.3	Machine is tested, faults are identified, and corrective action is taken.

Outcome 2

Sew dissimilar fabrics together.

Performance criteria

2.1	Work is completed to workplace performance criteria and in accordance with workplace procedures.

2.2	Faulty work is identified in accordance with workplace procedures.

2.3	Ply sequence is maintained.

2.4	Documentation is completed in accordance with workplace procedures.

| Replacement information | This unit standard and unit standard 2747 were replaced by skill standard 40135. |
| --- | --- |

This unit standard is expiring.  Assessment against the standard must take place by the last date for assessment set out below.

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 29 March 1995 | 31 December 2026 |
| Review | 2 | 10 February 1998 | 31 December 2026 |
| Revision | 3 | 10 October 2001 | 31 December 2026 |
| Revision | 4 | 16 June 2005 | 31 December 2026 |
| Review | 5 | 19 June 2009 | 31 December 2026 |
| Review | 6 | 29 August 2024 | 31 December 2026 |

| Consent and Moderation Requirements (CMR) reference | 0030 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.