| Title | Operate a tractor with an attached implement on rolling terrain under limited supervision |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 4 |

| Purpose | This unit standard is for people working under limited supervision with responsibility for operating a tractor with an attached implement to carry out work routines, in a primary industry context. People credited with this unit standard are able to operate a tractor with an attached implement on rolling terrain. |
| --- | --- |

| Classification | Agriculture > Agricultural Vehicles and Machinery |
| --- | --- |

| Prerequisites | Unit 31913, Operate a basic tractor on flat terrain under close supervision; and Unit 31914, Attach and detach a power take off driven 3 point linkage mounted implement to a basic tractor under close supervision; or demonstrate equivalent knowledge and skills. |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Legislation and regulations relevant to this unit standard includes but are not limited to the:

- Health and Safety at Work Act 2015;

- Land Transport (Driver Licensing) Rule 1999;

- Land Transport (Road User) Rule 2004.

- Definition

Limited supervision refers to the operation of a tractor with an attached implement under the limited guidance of a supervisor who has the knowledge and experience to ensure that the driver is safe at all times.

3	For the training and assessment of competency, operation of a tractor with an attached implement must not exceed a 20o slope.

4	Assessment

- During assessment, manufacturer’s recommendations and legislative requirements for the minimum age of a person driving a tractor must be complied with.

- Tractors must be over 1000kg and have a 3 point linkage.

- Assessment should consider the candidate’s behaviour while operating a tractor.  Behaviour is defined as a willingness to comply with, and consistent awareness of, safety requirements and workplace procedures.  Examples include; wearing appropriate footwear, operating the vehicle in a manner which minimises risk to self and others, avoiding ‘no go’ areas, not carrying passengers, and following instructions regarding care and maintenance of vehicles.

5	For the purposes of this unit standard, rolling terrain is defined as class D land under the Land Use Capability Classification, which comprises slopes between 16º and 20o (Rolling to strongly rolling). More detail is available from https://www.landcareresearch.co.nz/__data/assets/pdf_file/0017/50048/luc_handbook.pdf.

6	References

- Information on tractor safety is available from tractor manufacturers.

- NZTA, Agricultural Vehicles Guide: A guide for their safe and legal use on New Zealand roads (2017). Available from New Zealand Transport Agency website www.nzta.govt.nz.

- Accident Compensation Corporation www.acc.co.nz.

- Worksafe New Zealand www.worksafe.govt.nz.

7	Range

Implements may include but are not limited to – feed out wagon, power harrow, mower, silage wagon, tyne harrow, front end loader.

Outcomes and performance criteria

Outcome 1

Operate a tractor with an attached implement on rolling terrain under limited supervision.

Performance criteria

1.1	Identify hazards in operating a quad bike and describe the potential risks of harm to people.

Range	evidence is required for at least five hazards.

  - Identify control measures to eliminate or minimise health and safety risks.

1.3	Operate tractor with attached implement in a manner which suits the terrain, ground conditions, and implement in accordance with manufacturer’s specifications.

1.4	Select the gear prior to manoeuvre according suitability for the terrain and ground conditions.

Range	ground conditions include at least one of – wet and greasy surface, cultivated land, green and lush pasture, uphill, downhill, hard ground with heavy dew;

evidence of five difficult situations is required.

1.5	Apply tractor controls in accordance with manufacturer’s specifications.

1.6	Drive the tractor and attached implement at all times appropriate to the conditions, minimising risk of damage to self, others, property, and ground surfaces.

1.7	Report and/or record accidents or near misses.

1.8	Follow instructions and consistently demonstrate safe riding that complies with safety requirements.

| Replacement information | This unit standard replaced unit standard 24536 and unit standard 24537. |
| --- | --- |

| Planned review date | 31 December 2024 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 21 June 2012 | 31 December 2021 |
| Review | 2 | 24 October 2019 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0052 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact the Primary Industry Training Organisation standards@primaryito.ac.nz if you wish to suggest changes to the content of this unit standard.