| Title | Operate manual stairs for an aircraft |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 3 |

| Purpose | People credited with this unit standard are able to: carry out pre-checks and prepare manual stairs; set up and position manual stairs at aircraft door; and prepare and remove manual stairs from an aircraft. |
| --- | --- |

| Classification | Aviation > Ramp Operations |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

- All tasks are to be carried out in accordance with enterprise procedures, the enterprise being the organisation carrying out the work.  Enterprise procedures referred to in this unit standard are the applicable procedures found in the following: enterprise exposition, manufacturer publications, Government and local body legislation, and/or airworthiness authority requirements.

2	Definition

Manual stairs could refer to passenger boarding ramp used by different organisations.

3	Assessment

For an arriving aircraft, the following safety standards must be adhered to by ramp staff: anti-collision lights must be extinguished, aircraft wheels chocked, and engines shutdown, before the manual stairs are manoeuvred towards the aircraft and passenger door opened.

For a departing aircraft, the following safety standard must be adhered to by ramp staff: the aircraft passenger door must be closed, where applicable, before the manual stairs are manoeuvred away from the aircraft.

Outcomes and performance criteria

Outcome 1

Carry out pre-checks and prepare manual stairs.

Performance criteria

  - Protective clothing and safety equipment are worn.

Range	may include but is not limited to – high-visibility jacket, ear defenders.

  - Area of operation is checked for obstructions, and cleared if required.

  - Manual stairs are selected and prepared.

Range	may include but is not limited to – aircraft type, correct aircraft door, internal or external opening aircraft door, arrival gate, arrival time.

  - Manual stairs serviceability is checked prior to use.

Range	may include but is not limited to – stabilisers up, foreign object debris (FOD) on stairs or upper platform, safety strap in position, any other damage.

Outcome 2

Set up and position manual stairs at aircraft door.

Performance criteria

2.1	Manual stairs are pushed to stand-off area adjacent to the aircraft.

2.2	Manual stairs are pushed inside circle of safety.

2.3	Manual stairs are pushed to aircraft and positioned at aircraft door.

2.4	Platform buffer is aligned.

Range	may include but is not limited to – parallel to sill of aircraft door and not touching, fully engage stabilisers, undo safety strap.

Outcome 3

Prepare and remove manual stairs from an aircraft.

Performance criteria

3.1	Manual stairs are prepared for removal from the aircraft.

Range	may include but is not limited to – safety strap secured, FOD removed, retract stabilisers.

3.2	Manual stairs are pushed away from the aircraft and into a designated area.

| Planned review date | 31 December 2027 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 17 May 2012 | 31 December 2018 |
| Review | 2 | 16 March 2017 | 31 December 2024 |
| Revision | 3 | 25 July 2019 | 31 December 2024 |
| Review | 4 | 27 October 2022 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0125 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Ringa Hora Services Workforce Development Council qualifications@ringahora.nz if you wish to suggest changes to the content of this unit standard.