| Title | Demonstrate and apply knowledge of advanced control loop methods |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 7 |

| Purpose | People credited with this unit standard are able to: – demonstrate knowledge of advanced control loop methods; – configure advanced control loops; and – tune advanced control loops. |
| --- | --- |

| Classification | Industrial Measurement and Control > Industrial Measurement and Control - Theory |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

None.

Outcomes and performance criteria

Outcome 1

Demonstrate knowledge of advanced control loop methods.

Performance criteria

1.1	Describe limitations of single loop control.

Range	no load correction, no supply correction.

1.2	Describe principles and application of cascade control.

Range	primary loop, secondary loop 5 to 10 times faster;

inner loop types – valve positioner, flow, temperature (fast), pressure.

1.3	Describe application and principles of ratio control.

Range	mixing and blending, multiplier module, feedback trim.

1.4	Describe application and principles of feed-forward control.

Range	demand compensation, improved control loop response;

feed-forward element – flow, pressure, temperature.

1.5	Describe application and principles of split range control.

Range	two control valves, heating and cooling applications.

1.6	Describe application and principles of auto-selector control.

Range	select high, select low.

1.7	Describe application and principles of adaptive control.

Range	pre-programmed adaptive, self-adaptive, self-tuning.

1.8	Describe boiler feed-water control principles.

Range	single element, two element, three element, temperature and pressure compensation, shrink-swell dynamics of the drum.

Outcome 2

Configure advanced control loops.

Performance criteria

2.1	Configure cascade control loops and make operational.

Range	controller action (primary and secondary), element selection, transmitters, correct interconnection of controller components.

2.2	Configure feed-forward control loops and make operational.

Range	transmitters, feed-forward element selection, controller action, summer variables, correct interconnection of controller components.

2.3	Configure ratio control loops and make operational.

Range	transmitters, linear, square root controller inputs, controller action, correct interconnection of controller components.

2.4	Locate, interpret, and apply technical information to maintaining equipment.

Range	transmitters, controllers.

Outcome 3

Tune advanced control loops.

Performance criteria

3.1	Identify tuning and adjustment methods to be applied to each type of control loop.

Range	initial controller settings, systematic trial and error, ultimate sensitivity, reaction curve.

3.2	Tune cascade control loops and interpret and document resulting chart records.

Range	inner (secondary) loop first, outer (primary) loop second.

3.3	Tune ratio control loops and interpret and document resulting chart records.

3.4	Tune feed-forward control loops and interpret and document resulting chart records.

3.5	Apply personal and plant safety precautions.

Range	pressure, temperature, chemicals.

| Replacement information | This unit standard replaced unit standard 2660 and unit standard 2661. |
| --- | --- |

This unit standard is expiring.  Assessment against the standard must take place by the last date for assessment set out below.

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 28 November 2013 | 31 December 2027 |
| Rollover and Revision | 2 | 28 June 2018 | 31 December 2027 |
| Review | 3 | 30 January 2025 | 31 December 2027 |

| Consent and Moderation Requirements (CMR) reference | 0003 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.