| Title | Demonstrate knowledge of a diesel engine braking system and an exhaust brake |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 4 |

| Purpose | People credited with this unit standard are able to demonstrate knowledge of a diesel engine braking system, and an exhaust brake. |
| --- | --- |

| Classification | Motor Industry > Engines |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

- It is recommended that people hold credit for Unit 30477, Demonstrate knowledge of petrol and diesel engines before being assessed against this unit standard.

- Evidence presented for assessment against this unit standard must be consistent with safe working practices and be in accordance with applicable service information, company requirements and legislative requirements.  This includes the knowledge and use of suitable tools and equipment.

- Legislation, regulations and/or industry standards relevant to this unit standard include but are not limited to the current version of the Health and Safety at Work Act 2015 and any subsequent amendments or replacements.

- Definitions

Company requirements refers to instructions to staff on policy and procedures that are available in the workplace.  These requirements may include – company policies and procedures, work instructions, product quality specifications and legislative requirements.

Service information may include – technical information for a vehicle, machine, or product detailing operation; installation and servicing procedures; manufacturer instructions; technical terms and descriptions; and detailed illustrations.

Outcomes and performance criteria

Outcome 1

Demonstrate knowledge of a diesel engine braking system.

Performance criteria

  - The components of an engine braking system are identified.

1.2	The operation of an engine braking system is described.

Outcome 2

Demonstrate knowledge of an exhaust brake.

Performance criteria

2.1	The components of an exhaust brake are identified.

2.2	The operation of an exhaust brake is described.

| Planned review date | 31 December 2023 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 26 July 2018 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0014 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact MITO New Zealand Incorporated info@mito.org.nz if you wish to suggest changes to the content of this unit standard.