| Title | Induct users in the operation and inspection of suspended scaffolds |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 5 |

| Purpose | People credited with this unit standard are able to: - induct users in the safe use of suspended scaffolds; - induct users in the emergency procedures for suspended scaffolds; - induct users to complete daily suspended scaffold checks, explain the load test and check that the user induction procedures are conducted; - complete induction documentation to ensure the user understands safe operation, daily check procedures and the handover certificate procedures. |
| --- | --- |

| Classification | Lifting Equipment > Suspended Scaffolding |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	This unit standard has been developed for learning and assessment on-job or off-job in a simulated environment.

2	This unit is assessed in conjunction with Unit 26595, Complete suspended scaffolding inspection compliance reports.

3	All tasks must be carried out in accordance with:

a	quality management systems;

b	designer’s requirements and manufacturers' operating instructions; and legislation, regulations, bylaws, Health and Safety at Work Act 2015, and Health and Safety in Employment Regulations 1995;

c	the most up to date version of the Good Practice Guidelines - Scaffolding in New Zealand (GPG), 2016 available from

https://www.worksafe.govt.nz/topic-and-industry/working-at-height/scaffolding-in-new-zealand;

and all subsequent amendments and replacements.

4	Definitions

Suspended scaffolding is as defined in the GPG and in the Health and Safety in Employment Regulations 1995.

Suspended scaffolding must be supplied with clear instructions about safe configurations for installation and the intended uses, load combinations, pre-start checks and emergency procedures.

5	Training and assessment

This unit standard must be assessed against using proprietary suspended scaffolding systems and components.  Evidence is required for at least two suspended scaffolds, including a boatswain’s chair and a swinging stage.  Assessment must not be conducted using scaffolding which requires a Chartered Professional Engineer to certify the adequacy of the design prior to its erection.

6	This unit standard is based on a Suspended Scaffolding Inspection Compliance Report against the requirements of the GPG.  The focus for competency is the completion of the inspection report and therefore assessment may be conducted using any appropriate suspended scaffolding.  This scaffolding does not necessarily have to be erected by the candidate.

Outcomes and performance criteria

Outcome 1

Induct users in the safe use of suspended scaffolds.

Performance criteria

  - Explain the safe use of suspended scaffolds to users.

Range	safe access and egress from the suspended scaffold, safety of others in the vicinity, safe operation of suspended scaffold controls, security of power supply.

1.2	Induct users into safe operation, daily checks and emergency procedures for a suspended scaffold installation.

Outcome 2

Induct users in the emergency procedures for suspended scaffolds.

Performance criteria

2.1	Explain emergency procedures for a rescue from a suspended scaffold.

Range	evidence of minimum three is required, one of which is a non-powered descent of the suspended scaffold, one of use of emergency services.

Outcome 3

Induct users to complete daily suspended scaffold checks, explain the load test and check that the user induction procedures are conducted.

Performance criteria

3.1	Check the suspension rig.

Range	parapet brackets, needles and weights, suspension cables, supporting structure.

3.2	Check the cradle.

Range	hoist, brake mechanism, emergency mechanisms, fall arrest equipment, power supply.

3.3	Demonstrate and explain load test.

3.4	Check that the user induction procedures are conducted for suspended scaffolding.

Outcome 4

Complete induction documentation to ensure the user demonstrates safe operation, daily check procedures and the hand over certificate procedures.

Performance criteria

4.1	Demonstrate and explain the completion of a daily risk assessment for use of the suspended scaffold.

4.2	Demonstrate and explain the completion of the daily inspection report.

4.3	Demonstrate induction procedures and explain the completion of the suspended scaffold hand over certificate to the user.

Range	scaffold tags, handover certificates, inductions forms, manufacturer’s information.

| Planned review date | 31 December 2026 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 24 February 2022 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0003 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Waihanga Ara Rau Construction and Infrastructure Workforce Development Council qualifications@waihanga.nz if you wish to suggest changes to the content of this unit standard.