| Title | Set up and operate gluing machine, produce loose leaf binders, and quarter flush books for bookbinding |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 8 |

| Purpose | People credited with this unit standard are able to: check documentation and confirm requirements for the job are available; select and use adhesives; set up and operate a gluing machine for bookbinding; and produce loose leaf binders using fittings, and quarter flush books. |
| --- | --- |

| Classification | Printing > Binding and Finishing |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Legislation

Health and Safety at Work Act 2015.

2	Definitions

Accepted industry practice – approved codes of practice and standardised procedures accepted by the wider print industry as examples of best practice.

Job documentation refers to the documentation that is used in the workplace that contains the instructions and requirements for a particular production job.  This may include but is not limited to – workplace orders, production orders, workplace specifications, samples, lay cards, components and equipment required.

Job requirements refer to specific requirements for the job at hand.  These requirements may or may not be covered in the job documentation and may include special instructions, quality requirements expected by the customer, and/or production standards as set down by the workplace.

Workplace procedures refer to documented policies and procedures set by the organisation carrying out the work, and to documented or other directions provided to staff, and applicable to the tasks being carried out.  They may include but are not limited to – standard operating procedures, site specific procedures, site safety procedures, equipment operating procedures, quality assurance procedures, product quality specifications, references, approved codes of practice, housekeeping standards, environmental considerations, on-site briefings, supervisor’s instructions, and procedures to comply with legislative and local body requirements relevant to the industry sector.

3	Assessment information

All activities and evidence must meet workplace procedures, accepted industry practice, job documentation, and job requirements.

Outcomes and performance criteria

Outcome 1

Check documentation and confirm requirements for the job are available.

Performance criteria

1.1	Job documentation is checked to ensure that all specifications for the process being undertaken are complete and any discrepancies are reported.

1.2	Availability of components is confirmed.

1.3	Availability of equipment is confirmed.

Outcome 2

Select and use adhesives.

Performance criteria

2.1	Factors to be considered in selecting adhesives are described.

Range	may include but is not limited to – substrate, flexibility, temperature, working time, viscosity, machine or hand application, strength, acidity, speed of drying;

includes a minimum of two of – life, humidity, acidity, previous printing processes, compatibility to substrate and production process.

2.2	Adhesives are selected and used to suit the substrate.

Outcome 3

Set up and operate a gluing machine for bookbinding.

Performance criteria

3.1	Gluing machine used is described in terms of its functions.

Range	cold application, hot application.

3.2	Gluing machine is set up and operated.

3.3	Gluing machine is washed up and ready for next use.

3.4	Waste control procedures are followed.

Range	recyclable waste, non-recyclable waste, security waste.

Outcome 4

Produce loose leaf binders using fittings, and quarter flush books.

Performance criteria

4.1	Different types of fittings used for producing loose leaf binders are described.

Range	may include but is not limited to – inter-screws, split multi ring.

4.2	Loose leaf binders are produced using fittings.

4.3	Quarter flush and quarter flush sides turned-in books are produced.

| Planned review date | 31 December 2024 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 16 February 1995 | 31 December 2011 |
| Revision | 2 | 20 July 1998 | 31 December 2011 |
| Revision | 3 | 20 September 1999 | 31 December 2011 |
| Review | 4 | 30 November 2000 | 31 December 2011 |
| Review | 5 | 26 January 2005 | 31 December 2014 |
| Review | 6 | 12 December 2008 | 31 December 2015 |
| Review | 7 | 19 July 2012 | N/A |
| Review | 8 | 26 March 2020 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0013 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Competenz qualifications@competenz.org.nz if you wish to suggest changes to the content of this unit standard.