| Title | Set up and operate a collator or gatherer, and adhesive binding machine for print finishing |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 35 |

| Purpose | People credited with this unit standard are able to: check documentation and confirm requirements for the job are available; use signature recognition or spine marks for print finishing; set up a collator or gatherer, auxiliary equipment, and an adhesive binding machine for print finishing; and operate a collator or gatherer, and adhesive binding machine for print finishing. |
| --- | --- |

| Classification | Printing > Binding and Finishing |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Legislation

Health and Safety at Work Act 2015.

2	Definitions

Accepted industry practice – approved codes of practice and standardised procedures accepted by the wider print industry as examples of best practice.

Job documentation refers to the documentation that is used in the workplace that contains the instructions and requirements for a particular production job.  This may include but is not limited to – workplace orders, production orders, workplace specifications, samples, lay cards, components and equipment required.

Job requirements refer to specific requirements for the job at hand.  These requirements may or may not be covered in the job documentation and may include special instructions, quality requirements expected by the customer, and/or production standards as set down by the workplace.

Workplace procedures refer to documented policies and procedures set by the organisation carrying out the work, and to documented or other directions provided to staff, and applicable to the tasks being carried out.  They may include but are not limited to – standard operating procedures, site specific procedures, site safety procedures, equipment operating procedures, quality assurance procedures, product quality specifications, references, approved codes of practice, housekeeping standards, environmental considerations, on-site briefings, supervisor’s instructions, and procedures to comply with legislative and local body requirements relevant to the industry sector.

3	Assessment information

All activities and evidence must meet workplace procedures, accepted industry practice, job documentation, and job requirements.

Outcomes and performance criteria

Outcome 1

Check documentation and confirm requirements for the job are available.

Performance criteria

1.1	Job documentation is checked to ensure that all specifications for the process being undertaken are complete and any discrepancies are reported.

1.2	Availability of components is confirmed.

1.3	Availability of equipment is confirmed.

1.4	Sample copies of the job are hand assembled and any necessary changes are made.

Outcome 2

Use signature recognition or spine marks for print finishing.

Performance criteria

2.1	Use of signature recognition or spine marks are demonstrated to ensure that correct sequences are achieved.

Outcome 3

Set up a collator or gatherer, and auxiliary equipment for print finishing.

Range	includes jobs of at least four sections and cover.

Performance criteria

3.1	Collator or gatherer is set up and hoppers are timed to collating chain.

3.2	Auxiliary equipment is set up.

Range	may include but is not limited to – cover folder feeder, reply card attachments, tippers.

3.3	Crew assisting on collator or gatherer is supervised.

Outcome 4

Set up an adhesive binding machine for print finishing.

Range	binding methods may include but not limited to – notch bound, perfect bound, poly urethane resin (PUR) bound, sewn;

evidence of two binding methods are required;

jobs must include adhesive binding of at least four sections and cover.

Performance criteria

4.1	Factors to be considered in selecting adhesives are described.

Range	may include but is not limited to – substrate, flexibility, temperature, working time, viscosity, machine or hand application, strength, acidity, speed of drying;

a minimum of two of – life, humidity, acidity, previous printing processes, compatibility to substrate and production process.

4.2	Milling and roughing knives are set as required to suit the substrate.

4.3	Adhesives suitable for the substrate are selected and used.

4.4	Set up adhesive binding machine.

Range	adhesive supply system set including pre-melt tank, spine, side glue and reverse spinner are set, cover feeder, creasing, and nipping units are set.

4.5 	Delivery belt is set to meet requirements for speed, product thickness, and next process.

Outcome 5

Operate a collator or gatherer, and adhesive binding machine for print finishing.

Range	notch bound, perfect bound, poly urethane resin (PUR) bound, sewn;

evidence of two binding methods are required and jobs must include adhesive binding of at least four sections and cover.

Performance criteria

5.1	Collator or gatherer, and adhesive binding machine is operated.

Range	production speed, required quality, safety.

5.2	Crew assisting on equipment is supervised.

5.3	Jobs are prepared for the next process.

Range	knocked up, stacked, spines square, 3-knife trimmer requirements.

5.4	Adhesive binding machine is cleaned.

5.5	Waste control procedures are followed.

Range	recyclable waste, non-recyclable waste, security waste.

| Planned review date | 31 December 2024 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 16 February 1995 | 31 December 2011 |
| Revision | 2 | 20 July 1998 | 31 December 2011 |
| Revision | 3 | 20 September 1999 | 31 December 2011 |
| Review | 4 | 30 November 2000 | 31 December 2014 |
| Review | 5 | 26 January 2005 | 31 December 2015 |
| Review | 6 | 19 July 2012 | N/A |
| Review | 7 | 26 March 2020 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0013 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Competenz qualifications@competenz.org.nz if you wish to suggest changes to the content of this unit standard.