| Title | Use a cab controlled gantry crane to lift and place regular loads |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 2 |

| Purpose | People credited with this unit standard are able to: – demonstrate knowledge of using a cab controlled gantry crane; – use a cab controlled gantry crane to lift and place regular loads; and – carry out daily and weekly operator maintenance of a cab controlled gantry crane. |
| --- | --- |

| Classification | Cranes > Crane Operation |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

| Prerequisite | Unit 3800, Use a radio remote or pendant controlled gantry crane to lift and place regular loads, or demonstrate equivalent knowledge and skills. |
| --- | --- |

Guidance Information

1	Definitions

Crane – the definition as given in the Health and Safety in Employment (Pressure Equipment, Cranes, and Passenger Ropeways) Regulations 1999.

Gantry crane – one of a group of cranes collectively known as gantry cranes that generally have an overhead beam as a main part of their structure.

Industry good practice – practice that meets the requirements of the Approved Code of Practice for Cranes, Approved Code of Practice for Load-lifting Rigging, and the Crane Safety Manual.

Regular loads have the following characteristics – uniform weight distribution; concentric loading or regular proportions; known lifting points; repetitively lifted.

2	Assessment

- Assessment must be carried out in a crane workplace or other non-simulated environment.  Non-simulated environment means; working with the skill sets required in authentic experience in an organisational context, with commercial competence and the expectations and possible consequences of that context.

- Assessment must involve a minimum of two different assessor observed lifts.

3	All tasks are to be carried out in accordance with the equipment manufacturer requirements, applicable company procedures, and industry good practice found in the following sources:

a	company quality management systems;

b	requirements and guidelines consistent with the Health and Safety at Work Act 2015;

c	equipment manufacturer operating instructions;

d	government and local government legislation, regulations, and bylaws;

e	Crane Safety Manual, version 3 or higher, available from http://shop.cranes.org.nz/;

f	Approved Code of Practice for Cranes available from http://www.business.govt.nz/worksafe/information-guidance/all-guidance-items.

Outcomes and performance criteria

Outcome 1

Demonstrate knowledge of using a cab controlled gantry crane.

Performance criteria

1.1	Demonstrate knowledge of the operation of, and hazards associated with, a cab controlled gantry crane.

Range	may include – twin hoist or twin gantry for specific workplaces that have these available.

Outcome 2

Use a cab controlled gantry crane to lift and place regular loads.

Performance criteria

2.1	Identify and implement controls to lower any identified risks, and foreseeable hazards.

Range	may include but is not limited to – access, obstructions, field of vision, travel route, unauthorised people, insecure loads, unstable loads, unknown loads, type of load.

2.2	Confirm any person directing lifts, and the communication methods to be used during lift operation.

2.3	Confirm load has been slung.

2.4	Access and operate a cab controlled gantry crane to lift, travel, and unload or place regular loads in response to recognised overhead crane hand signals and radio communication.

Outcome 3

Carry out daily and weekly operator maintenance of a cab controlled gantry crane.

Performance criteria

3.1	Carry out daily and weekly crane checks and maintenance.

| Planned review date | 31 December 2022 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 9 April 1995 | 30 June 2013 |
| Review | 2 | 22 May 1997 | 30 June 2013 |
| Review | 3 | 27 March 2000 | 30 June 2013 |
| Review | 4 | 20 June 2006 | 30 June 2013 |
| Review | 5 | 15 March 2012 | 31 December 2018 |
| Review | 6 | 28 September 2017 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0003 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact The Skills Organisation reviewcomments@skills.org.nz if you wish to suggest changes to the content of this unit standard.