| Title | Configure and position a mobile crane, and lift and place regular and irregular loads |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 25 |

| Purpose | People credited with this unit standard are able to: – demonstrate knowledge of using a mobile crane; – configure and position a mobile crane; – lift and place regular and irregular loads with a mobile crane; – prepare mobile crane for travel mode, and complete documentation; and – carry out daily and weekly operator maintenance of a mobile crane. |
| --- | --- |

| Classification | Cranes > Crane Operation |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

| Prerequisite | Unit 3789, Sling varied regular loads and safely direct a crane during crane operations, or demonstrate equivalent knowledge and skills. |
| --- | --- |

Guidance Information

1	All tasks are to be carried out in accordance with the equipment manufacturer requirements, applicable company procedures, and industry good practice found in the following sources:

a	company quality management systems;

b	health and safety requirements and guidelines consistent with the Health and Safety at Work Act 2015;

c	equipment manufacturer operating instructions;

d	government and local government legislation, regulations, and bylaws;

e	Crane Safety Manual, version 3.2 or higher, available from http://shop.cranes.org.nz/;

f	Approved Code of Practice for Cranes, 2009 (3rd edition), and/or the Approved Code of Practice for Load-lifting Rigging available from http://www.business.govt.nz/worksafe/information-guidance/all-guidance-items.

2	Definitions

Crane – the definition as given in the Health and Safety in Employment (Pressure Equipment, Cranes, and Passenger Ropeways) Regulations 1999.

Crane Workplace – the definition as given in Health and Safety at Work Act 2015.

Irregular loads have one or more of the following characteristics – unequal weight distribution, eccentric loading, irregular shape and proportions, with or without set lifting points.

Industry good practice – practice that meets the requirements of the Approved Code of Practice for Cranes, Approved Code of Practice for Load-lifting Rigging, and the Crane Safety Manual.

Lift plan – the planning of a lift that may be in a variety of formats, in accordance with company procedures.

Regular loads have the following characteristics – uniform weight distribution; concentric loading or regular proportions; known lifting points; repetitively lifted.

Roading the crane – includes understanding or demonstration of Certificate of fitness /Warrant of fitness, Registration applicable permits (national and regional road controlling authorities), swept path requirements, license and BESS requirements (Bridge engineering self supervision), crane configuration, driving on road.

Risk assessment – may include communication and documentation of a lift plan, toolbox talk, task analysis, or job safety analysis.

3	Assessment

- Cranes that can be used for assessment against this unit standard include – truck mounted cranes, all terrain cranes, rough terrain cranes.

- Assessment cannot be carried out using truck loader cranes, or telehandlers, or excavators when being used as cranes.  Assessment for competency in the use of truck loader cranes is covered by Unit 16617, Use a truck loader crane to lift and place regular loads.

- Assessment must be carried out in a crane workplace or other non-simulated environment.  Non-simulated environment means; working with the skill sets required in authentic experience in an organisational context, with commercial competence and the expectations and possible consequences of that context.

- Evidence used for assessment against this unit standard must meet one or more of the following requirements to prove experience:

  - verified experiences from authorised verifier documented and observed lifts, or company documents with a minimum of 40 lifts (slinging and directing placement of load) with at least six variations of lifting operations that may include site, crane, loads, and communication methods; and

  - at least two irregular loads, including one in which centre of gravity is offset, must be observed by the assessor.

4	It is recommended that Unit 31904, Plan and document lifts of regular and irregular loads using multiple cranes is completed prior to assessment against this unit standard.

Outcomes and performance criteria

Outcome 1

Demonstrate knowledge of using a mobile crane.

Performance criteria

1.1	Describe hazards associated with the operation of a mobile crane.

Range	includes – hazards associated with roading the crane.

1.2	Describe the roading and traveling of a mobile crane to a workplace.

Range	includes but is not limited to – assembling and checking required equipment, authorisation to use equipment, pre-start checks and service, fault reporting, roading the crane.

1.3	Describe the setting up of a mobile crane on site.

Range	includes but is not limited to – site arrival and induction processes, permit to work, access and exiting, crane platform assessment, risk assessment, establishing exclusion zones.

1.4	Explain the operation of a mobile crane.

Range	includes but is not limited to – rating charts and range diagrams.

1.5	Explain the requirements for lock out and tag out processes for the mobile crane.

Outcome 2

Configure and position a mobile crane.

Performance criteria

2.1	Analyse task to identify regulatory requirements and permits that are needed in accordance with the lift plan, task, and site.

Range	may include but is not limited to – Police, New Zealand Transport Agency, local body requirements, Corridor Access Request (CAR), electricity supply, telecommunications, Worksafe New Zealand, Civil Aviation Authority, Rail Corridor Requirements.

2.2	Analyse the workplace to identify and document hazards and risks, relating to the planned lift, and implement control measures so far as reasonably practicable.

Range	may include but is not limited to – vehicles, equipment, ground condition, underground services, overhead power lines, overhead service lines, trees, buildings, structures, unauthorised people, exclusion zones, wind and weather, simultaneous activities.

2.3	Configure and position the crane in accordance with the lift plan.

Range	includes – configuration of blocks, outriggers, safe load indicator, boom length, anti-two-block, and establishing weight of load (before lift);

may also include configuration of – counterweight, operating limits, fly jib.

2.4	Configure ropes, reeving and parts of line, hook block, hook, and rigging equipment for the load to be lifted.

2.5	Carry out plant and equipment operational and safety checks.

Range	may include but is not limited to – settlement test.

Outcome 3

Lift and place regular and irregular loads with a mobile crane.

Performance criteria

3.1	Communicate the tasks, hazards, roles, and responsibilities to workers assisting with the lift and other persons who may be affected, in accordance with the lift plan.

3.2	Check to ensure workers assisting with the lift and other persons who may be affected are in their respective positions ready to conduct their tasks.

Range	may include but is not limited to – riggers, slingers, dogmen, supervisor.

3.3	Observe and check to confirm that the load is correctly attached, and lift and place the load in accordance with the lift plan and customer requirements.

Range	includes – regular and irregular loads.

Outcome 4

Prepare mobile crane for travel mode, and complete documentation.

Performance criteria

4.1	Prepare mobile crane for travel and store lifting and other equipment used in conjunction with the lift.

Range	travel includes – on road and off road travel.

4.2	Secure boom in travel position.

Range	travel includes – on road and off road travel.

4.3	Record and take action for any defects in crane and equipment.

4.4	Complete documentation as required.

Range	includes but is not limited to – log books, records, job sheets, crane equipment inventories, maintenance requests.

Outcome 5

Carry out daily and weekly operator maintenance of a mobile crane.

Performance criteria

5.1	Inspect the crane and inform the crane controller of any issues that impact the operation of the crane in accordance with safety requirements.

| Planned review date | 31 December 2024 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 9 April 1995 | 30 June 2013 |
| Review | 2 | 22 May 1997 | 30 June 2013 |
| Review | 3 | 27 March 2000 | 30 June 2013 |
| Review | 4 | 20 June 2006 | 30 June 2013 |
| Revision | 5 | 26 March 2007 | 30 June 2013 |
| Review | 6 | 15 March 2012 | 31 December 2018 |
| Review | 7 | 28 September 2017 | 31 December 2021 |
| Review | 8 | 26 September 2019 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0003 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact The Skills Organisation reviewcomments@skills.org.nz if you wish to suggest changes to the content of this unit standard.