| Title | Install fence netting |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 8 |

| Purpose | People credited with this unit standard are able to: run out, join, and set up fence netting; tension and terminate fence netting; and staple fence netting. |
| --- | --- |

| Classification | Agriculture > Fencing |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Legislation relevant to this unit standard includes but is not limited to the Health and Safety at Work Act 2015.

2	All evidence presented in this unit standard must be in accordance with workplace procedures.

3	Definition

Workplace procedures refer to policies and procedures on safety and operation set down by the employer or host organisation and includes manufacturer’s guidelines for use of equipment and materials.

Outcomes and performance criteria

Outcome 1

Run out, join and set up fence netting.

Performance criteria

1.1	Roll out netting along entire fence line without damage and in a controlled manner.

1.2	Join netting ends with no protrusions.

1.3	Prepare netting with wider wire spacings at the top of the fence, allowing unrestricted straining and adjustment before termination.

1.4	Position termination knots according to assembly type to avoid slack.

Outcome 2

Tension and terminate fence netting.

Performance criteria

2.1	Place netting clamps on netting against stay wire, and secure to prevent slip or wire damage.

2.2	Complete the pre-termination line check prior to final tensioning.

2.3	Tension netting with no vertical distortion.

2.4	Terminate line wires and retain tension when netting clamps are removed.

Outcome 3

Staple fence netting.

Performance criteria

3.1	Position the netting height according to fence design.

3.2	Skew staples with minimum wire clearance and maintain true staple loading to all primary and intermediate posts.

3.3	Maintain ground clearance in a uniform manner.

3.4	Staple wire to post without damage in accordance with post loading.

3.5	Identify netting section requiring alteration and achieve the required adjustment.

| Planned review date | 31 December 2025 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 28 February 1993 | 31 December 2023 |
| Review | 2 | 29 September 1995 | 31 December 2023 |
| Review | 3 | 28 February 1997 | 31 December 2023 |
| Review | 4 | 25 June 2002 | 31 December 2023 |
| Review | 5 | 22 August 2008 | 31 December 2023 |
| Review | 6 | 29 April 2021 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0052 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact the Primary Industry Training Organisation standards@primaryito.ac.nz if you wish to suggest changes to the content of this unit standard.