| Title | Carry out aircraft ground handling |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 4 |

| Purpose | People credited with this unit standard are able to: marshal aircraft; assist with engine start-up and shutdown; move aircraft manually or by towing; and park and moor aircraft. |
| --- | --- |

| Classification | Aeronautical Engineering > Aircraft Servicing |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

- All tasks must be carried out in accordance with enterprise procedures.

- Definition

Enterprise procedures – procedures used by the organisation carrying out the work and applicable to the tasks being carried out.  Examples are – standard operating procedures, safety procedures, equipment operating procedures, codes of practice, quality management practices and standards, procedures to comply with legislative and local body requirements.

3	This standard must be assessed against in the workplace or by the use of simulated activities off-the-job.

4	The scope of the system that this standard relates to is described in ATA iSpec 2200, chapters 9 and 10.

Outcomes and performance criteria

Outcome 1

Marshal aircraft.

Performance criteria

1.1	Marshalling equipment, clothing, and assistants are selected and used.

1.2	Marshalling positions are selected to ensure adequate visibility and safety.

Range	for flight crew, for ground markings, for ground personnel, for aircraft clearance from obstructions.

1.3	Aircraft is marshalled using International Civil Aviation Organisation (ICAO), International Airline Transportation Association (IATA) or Air Force Interoperability Council (AFIC) marshalling signals.

Outcome 2

Assist with engine start-up and shutdown.

Performance criteria

2.1	Aircraft is prepared for engine start.

Range	may include but is not limited to – blanks, covers and screens are removed or fitted (as required); chocks and brakes are applied; aircraft is positioned; fire extinguisher is positioned; access doors are closed; area checked clear around the aircraft.

2.2	Start services are applied and removed.

2.3	Fires discovered on engine or services during start-up or shutdown, or brakes during shutdown are controlled and/or reported.

Range	pilot signalled, fire section notified, attempt to extinguish.

2.4	Communication with flight deck crew is established.

Range	may include – verbal, visual, electronic.

2.5	Emergency or precautionary engine shutdown procedures are followed.

2.6	Emergency shutdowns are recorded and reported.

Outcome 3

Move aircraft manually or by towing.

Performance criteria

3.1	Aircraft is prepared to be moved.

Range	may include but is not limited to – structural panels fitted; doors, panels and cowls closed; covers, blanks, locks, chocks and steering mechanisms positioned as required; clearance obtained.

3.2	Towing and moving equipment are selected and fitted, and personnel are selected and positioned.

Range	may include but is not limited to – tow bars, tow arms, bridles, wheels, towing vehicles;

personnel may include but are not limited to – clearance personnel, aircraft brake operator, tow vehicle driver.

3.3	Communication is established and maintained between aircraft, control tower (where applicable), towing vehicle, and clearance crew.

Range	may include – verbal, visual, electronic.

3.4	Aircraft is moved in accordance with aircraft type.

Outcome 4

Park and moor aircraft.

Performance criteria

4.1	Aircraft is parked.

4.2	Aircraft is tied down in accordance with aircraft type.

4.3	Securing equipment is installed and removed in accordance with aircraft type.

Range	may include but is not limited to – locks, chocks, covers, blanks, propeller or rotor tie downs, special equipment.

4.4	Documentation is completed.

| Planned review date | 31 December 2027 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 19 June 1995 | 31 December 2016 |
| Revision | 2 | 7 August 1997 | 31 December 2016 |
| Revision | 3 | 8 May 2001 | 31 December 2016 |
| Review | 4 | 20 April 2006 | 31 December 2016 |
| Review | 5 | 18 June 2014 | 31 December 2021 |
| Revision | 6 | 28 June 2018 | 31 December 2021 |
| Review | 7 | 26 March 2020 | N/A |
| Rollover and Revision | 8 | 26 April 2024 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0028 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Ringa Hora Services Workforce Development Council qualifications@ringahora.nz if you wish to suggest changes to the content of this unit standard.