| Title | Apply aeronautical decals |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 4 |

| Purpose | People credited with this unit standard are able to: apply decals to aircraft, aircraft components, or aeronautical equipment; and complete finishing activities. |
| --- | --- |

| Classification | Aeronautical Engineering > Aircraft Painting |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

- All tasks must be carried out in accordance with enterprise procedures.

- Definition

Enterprise procedures – procedures used by the organisation carrying out the work and applicable to the tasks being carried out.  Examples are – standard operating procedures, safety procedures, equipment operating procedures, codes of practice, quality management practices and standards, procedures to comply with legislative and local body requirements.

Outcomes and performance criteria

Outcome 1

- Apply decals to aircraft, aircraft components, or aeronautical equipment.

Performance criteria

1.1	Task is determined by reviewing maintenance documentation and enterprise procedures.

1.2	Resources are obtained and checked for serviceability or status.

Range	may include but are not limited to – tools, materials, equipment, safety equipment, clothing, publications.

1.3	Aircraft, component, or aeronautical equipment identification is matched with documentation.

1.4	Environment is established.

Range	temperature, lighting.

1.5	Ground equipment is positioned.

1.6	Decals are prepared.

1.7	Aircraft, component, or aeronautical equipment surface is prepared.

Range	cleaned, degreased.

1.8	Decals are applied.

Range	may include but is not limited to – measured, marked out, positioned, edge sealed.

1.9	Inspections are obtained.

Outcome 2

- Complete finishing activities.

Performance criteria

2.1	Resources are checked for serviceability.

2.2	Leftover parts and materials are disposed of.

Range	may include but is not limited to – serviceable, unserviceable, surplus, waste.

2.3	Documentation is completed.

2.4	Completion activities specific to the task and work area are carried out.

Range	may include but are not limited to – tool control, cleanliness, tidiness, return of publications, preparation for next activity, return of aircraft and systems to normal.

| Planned review date | 31 December 2027 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 19 June 1995 | 31 December 2016 |
| Revision | 2 | 7 August 1997 | 31 December 2016 |
| Revision | 3 | 8 May 2001 | 31 December 2016 |
| Review | 4 | 20 April 2006 | 31 December 2016 |
| Review | 5 | 18 June 2014 | 31 December 2021 |
| Review | 6 | 26 March 2020 | N/A |
| Rollover and Revision | 7 | 26 April 2024 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0028 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Ringa Hora Services Workforce Development Council qualifications@ringahora.nz if you wish to suggest changes to the content of this unit standard.