| Title | Shape aeronautical sheet metals by cutting |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 6 |

| Purpose | People credited with this unit standard are able to: prepare to shape aeronautical sheet metals by cutting; cut aeronautical sheet metals; and carry out task completion activities. |
| --- | --- |

| Classification | Aeronautical Engineering > Aircraft Structures |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	All tasks must be carried out in accordance with enterprise procedures.

2	Definition

Enterprise procedures – procedures used by the organisation carrying out the work and applicable to the tasks being carried out.  Examples are – standard operating procedures, safety procedures, equipment operating procedures, codes of practice, quality management practices and standards, procedures to comply with legislative and local body requirements.

Outcomes and performance criteria

Outcome 1

Prepare to shape aeronautical sheet metals by cutting.

Performance criteria

1.1	Task is determined by reviewing maintenance documentation and enterprise procedures.

1.2	Work area is prepared, and resources are obtained and checked for serviceability or status.

Range	may include but is not limited to – publications, tools, materials, heat treatment states, equipment, safety equipment.

1.3	Material is marked out to meet task requirements.

Range	parallax error avoided, material orientation, marking equipment and materials.

Outcome 2

Cut aeronautical sheet metals.

Performance criteria

2.1	Material is secured to prevent movement during cutting.

2.2	Material is cut to dimensions.

Range	using a minimum of ten of the following – guillotines, hole saws, hack saws, band saws, snips, nibblers, files, reamers, portable drills, hand punches or power punches, routers, drill presses.

2.3	Any defects are rectified.

2.4	Material is dressed to remove sharp edges and stress raisers.

2.5	Inspections are obtained.

Outcome 3

Carry out task completion activities.

Performance criteria

3.1	Part is prepared for use, storage, or transit.

3.2	Completion activities specific to the task and work area are carried out.

Range	may include but is not limited to – tool control, cleanliness, tidiness, return of publications, preparation for next activity, return of aircraft and systems to normal.

3.3	Leftover items, parts, and materials are disposed of.

Range	may include but is not limited to – serviceable, unserviceable, surplus, waste, scrap, hazardous.

3.4	Documentation is completed.

| Planned review date | 31 December 2027 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 19 June 1995 | 31 December 2016 |
| Revision | 2 | 7 August 1997 | 31 December 2016 |
| Revision | 3 | 8 May 2001 | 31 December 2016 |
| Review | 4 | 26 March 2007 | 31 December 2016 |
| Review | 5 | 18 June 2014 | 31 December 2021 |
| Review | 6 | 26 March 2020 | N/A |
| Rollover and Revision | 7 | 26 April 2024 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0028 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Ringa Hora Services Workforce Development Council qualifications@ringahora.nz if you wish to suggest changes to the content of this unit standard.