| Title | Form aeronautical sheet metals by shrinking and stretching |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 8 |

| Purpose | People credited with this unit standard are able to: prepare to form aeronautical sheet metals by shrinking and stretching; form aeronautical sheet metals by shrinking and stretching; and carry out task completion activities. |
| --- | --- |

| Classification | Aeronautical Engineering > Aircraft Structures |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	All tasks must be carried out in accordance with enterprise procedures.

2	Definition

Enterprise procedures – procedures used by the organisation carrying out the work and applicable to the tasks being carried out.  Examples are – standard operating procedures, safety procedures, equipment operating procedures, codes of practice, quality management practices and standards, procedures to comply with legislative and local body requirements.

Outcomes and performance criteria

Outcome 1

Prepare to form aeronautical sheet metals by shrinking and stretching.

Performance criteria

1.1	Task is determined by reviewing maintenance documentation and enterprise procedures.

1.2	Work area is prepared, and resources are obtained and checked for serviceability or status.

Range	may include but is not limited to – publications, tools, materials, heat treatment states, equipment, safety equipment.

1.3	Form blocks are manufactured to meet task requirements.

Outcome 2

Form aeronautical sheet metals by shrinking and stretching.

Performance criteria

2.1	Sheet metal is cut to meet specifications.

Range	mark out, cut, deburr.

2.2	Sheet metal is formed to specifications by shrinking and stretching.

2.3	Any defects are rectified.

2.4	Inspections are obtained.

Outcome 3

Carry out task completion activities.

Performance criteria

3.1	Item is prepared for use, storage, or transit.

3.2	Completion activities specific to the task and work area are carried out.

Range	may include but is not limited to – tool control, cleanliness, tidiness, return of publications, preparation for next activity, return of aircraft and systems to normal.

3.3	Leftover items, parts, and materials are disposed of.

Range	may include but is not limited to – serviceable, unserviceable, surplus, waste, scrap, hazardous.

3.4	Documentation is completed.

| Planned review date | 31 December 2027 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 19 June 1995 | 31 December 2016 |
| Revision | 2 | 7 August 1997 | 31 December 2016 |
| Revision | 3 | 8 May 2001 | 31 December 2016 |
| Review | 4 | 26 March 2007 | 31 December 2016 |
| Review | 5 | 18 June 2014 | 31 December 2021 |
| Review | 6 | 26 March 2020 | N/A |
| Rollover and Revision | 7 | 26 April 2024 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0028 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Ringa Hora Services Workforce Development Council qualifications@ringahora.nz if you wish to suggest changes to the content of this unit standard.