| Title | Spin yarn on a woollen ring spinning machine |  |  |
| --- | --- | --- | --- |
| Level | 2 | Credits | 8 |

| Purpose | People credited with this unit standard are able to: demonstrate knowledge of woollen spinning and the woollen spinning machine; prepare and operate a woollen spinning machine; identify and fix operational and product faults; and keep spinning machine and work area clean. |
| --- | --- |

| Classification | Textiles Manufacture > Woollen Yarn Processing |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Legislation relevant to this unit standard includes but is not limited to the:

Health and Safety at Work Act 2015.

2	Definitions
Spools refer to bobbins in some woollen spinning plants.  Spools contain the slubbing produced from the card.

Workplace procedures refer to the procedures used by the organisation carrying out the work and applicable to the tasks being carried out.  Examples are – standard operating procedures, safety procedures, equipment operating procedures, codes of practice, quality management practices and standards, procedures to comply with legislative and local body requirements.

3	Assessment information

a	All activities and evidence must be in accordance with workplace procedures.

b	Competence will be demonstrated on a woollen ring spinning machine used in 	the candidate’s workplace.

Outcomes and performance criteria

Outcome 1

Demonstrate knowledge of woollen spinning and the woollen spinning machine.

Performance criteria

1.1	Woollen spinning is described in terms of purpose and yarn characteristics.

1.2	Sections of a woollen spinning frame are described in terms of their function.

Range	may include – creel, drafting zone, false twist, spindles, ring and traveller, waste collection, headstock, machine controls, spindle tape, spindle brake, lappet, oiling device, waste recycling.

1.3	Spinning faults are identified and described in terms of significance and corrective action.

Range	may include – contamination, double ends, thick and thin yarn, twist variation, piecening faults, package shape, package density, oil and grease marks, excessive end breaks, traveller wear, light and heavy ends.

Outcome 2

Prepare and operate a woollen spinning machine.

Performance criteria

2.1	Personal protective equipment is used.

2.2	Safe work methods are demonstrated.

2.3	Machine is prepared for operation.

Range	batch identification, creel loaded, travellers fitted, thread up and run on, empty tubes in place.

2.4	Yarn is spun.

Range	piece up end breaks, pull back ends on spool, thread drafting zone, doff full yarn packages, change spool sets, maintain ring oil, adjust spool tension, fake twist revolutions per minute.

2.5	Product is handled and recorded.

Outcome 3

Identify and fix operational and product faults.

Performance criteria

3.1	Yarn faults are identified and rectified or reported.

Range	may include – contamination, double ends, thick and thin yarn, twist variation, piecening faults, package shape, package density, oil and grease marks, excessive end breaks, wear, piecening faults, light/heavy ends.

3.2	Spinning machine malfunction is identified and rectified or reported.

Outcome 4

Keep spinning machine and work area clean.

Performance criteria

4.1	Work area and equipment is kept clean during processing.

4.2	Spinning machine is cleaned at end of a batch.

4.3	Waste fibre is collected and sorted according to type to prevent contamination.

| Planned review date | 31 December 2026 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 18 June 1995 | 31 December 2023 |
| Revision | 2 | 8 August 1997 | 31 December 2023 |
| Revision | 3 | 18 July 2000 | 31 December 2023 |
| Revision | 4 | 10 October 2001 | 31 December 2023 |
| Revision | 5 | 12 August 2004 | 31 December 2023 |
| Review | 6 | 23 April 2008 | 31 December 2023 |
| Review | 7 | 27 January 2022 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0030 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact the Hanga-Aro-Rau Manufacturing, Engineering, and Logistics Workforce Development Council qualifications@hangaarorau.nz if you wish to suggest changes to the content of this unit standard.