| Title | Demonstrate knowledge of, and use, coordinate measuring machine (CMM) technology |  |  |
| --- | --- | --- | --- |
| Level | 5 | Credits | 6 |

| Purpose | People credited with this unit standard are able to demonstrate knowledge of CMM technology; take measurements using a coordinate measuring machine; and produce and evaluate data. |
| --- | --- |

| Classification | Mechanical Engineering > Engineering - Measurement |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Explanatory notes

1	References

Health and Safety in Employment Act 1992.

ISO 1101:2004, Geometrical product specifications (GPS) – Geometrical tolerancing – Tolerances of form, orientation, location and run-out.

ISO 10360-Parts 1-6, Geometrical Product Specifications (GPS) – Acceptance and reverification tests for coordinate measuring machines (CMM), (2000/2001).

2	Range

Use of CMMs in this unit standard requires the measurement of various geometric tolerancing characteristics that may include but are not limited to – straightness, flatness of an internal plane, circularity, profile of any line, parallelism, perpendicularity, angularity, position, coaxiality, skew of two cylinder axes, distance between two bores, sphericity, and total run-out.

Evidence of measurement of at least four different characteristics on at least three simple three dimensional (3D) workpieces is required.

Outcomes and evidence requirements

Outcome 1

Demonstrate knowledge of CMM technology.

Evidence requirements

1.1	CMMs are compared in terms of types available and their basic operating principles.

Range	may include but is not limited to – manual, computer numerical control (CNC) operated, horizontal arm, gantry, twin column, articulated arm, track mounted, software interface.

1.2	Machine components are described in terms of types available, and their function in the process of measuring.

Range	may include but is not limited to – wrist, stylus, probe (laser, contact, touch trigger), workpiece table, slides (xyz), rotary table (C-axis), camera/video probe.

1.3	CMM technology is described in terms of advantages and disadvantages compared with conventional metrology equipment.

Range	may include but is not limited to – time savings, cost, accuracy of measurements, elimination of multiplicity of expensive gauges, software complexities, training in use.

1.4	Terminology associated with CMM technology is explained.

Range	reverse engineering, surface or profile digitising, qualification of styli.

Outcome 2

Take measurements using a coordinate measuring machine.

Range	manual or CNC operated.

Evidence requirements

2.1	Volumetric accuracy of selected CMM is established prior to commencing task to ensure job specifications can be met.

2.2	Calibration certificate is viewed for the selected machine in advance of commencing the measurement task.

2.3	Coordinates and datum points are established, as appropriate, in accordance with machine type and job requirements.

2.4	Stylus selection is in accordance with the workpiece being measured and job requirements.

2.5	Measurement points are established relevant to the workpiece being measured, CMM type, and job requirements.

2.6	Workpiece is placed correctly within the machine to ensure ease of access to measurement points.

2.7	Measurements are confirmed through re-measurement at different points on the workpiece.

Outcome 3

Produce and evaluate data.

Evidence requirements

3.1	Raw data is produced in accordance with machine and software types.

Range	hard copy and electronic formats.

3.2	Data evaluation compares measurements with computer aided design (CAD) model or specifications, establishes measurement uncertainty, and takes into account ambient temperature variations.

| Planned review date | 31 December 2016 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 23 May 1995 | 31 December 2011 |
| Revision | 2 | 14 April 1997 | 31 December 2011 |
| Revision | 3 | 5 January 1999 | 31 December 2011 |
| Revision | 4 | 23 May 2001 | 31 December 2011 |
| Review | 5 | 21 February 2005 | 31 December 2014 |
| Review | 6 | 17 November 2011 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0013 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Please note

Providers must be granted consent to assess against standards (accredited) by NZQA, before they can report credits from assessment against unit standards or deliver courses of study leading to that assessment.

Industry Training Organisations must be granted consent to assess against standards by NZQA before they can register credits from assessment against unit standards.

Providers and Industry Training Organisations, which have been granted consent and which are assessing against unit standards must engage with the moderation system that applies to those standards.

Requirements for consent to assess and an outline of the moderation system that applies to this standard are outlined in the Consent and Moderation Requirements (CMR).  The CMR also includes useful information about special requirements for organisations wishing to develop education and training programmes, such as minimum qualifications for tutors and assessors, and special resource requirements.

Comments on this unit standard

Please contact Competenz on qualifications@competenz.org.nz if you wish to suggest changes to the content of this unit standard.