| Title | Paint vehicle panels in solid colour two pack in the automotive refinishing industry |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 8 |

| Purpose | This unit standard is for people who work in the automotive refinishing industry. People credited with this unit standard are able to check the preparation of vehicle panels, and apply solid colour two pack coats on prepared vehicle panels. |
| --- | --- |

| Classification | Motor Industry > Automotive Refinishing |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Legislation and references

Performance of the outcome of this unit standard must comply with the following:

Hazardous Substances and New Organisms Act 1996;

Health and Safety at Work Act 2015;

Spray Coating Regulations 1962;

Approved Code of Practice for the Safe Use of Isocyanates (WorkSafe).

2	Any new, amended or replacement Acts, regulations, standards, codes of practice, guidelines, or authority requirements or conditions affecting this unit standard will take precedence for assessment purposes, pending review of this unit standard.

3	Approved Code of Practice for the Safe Use of Isocyanates is available online from the WorkSafe website www.business.govt.nz/worksafe/information-guidance/all-guidance-items/acop-isocyanates.

4	Definitions

Company requirements refer to instructions to staff on policy and procedures which are documented in memo or manual format and are available in the workplace.  These requirements include but are not limited to – company specifications and procedures, work instructions, manufacturer specifications, product quality specifications and legislative requirements.

Heavy vehicle refers to a motor vehicle that is of Class MD3, MD4, ME, NB, NC, TC or TD; or has a gross vehicle mass that exceeds 3500 kg and is not of a class specified in the table of vehicle classes as listed from New Zealand Transport Agency website http://vehicleinspection.nzta.govt.nz/virms/in-service-wof-and-cof/introduction/inspection-and-certification-process/identifying-the-vehicle-class.

Light vehicle refers to classes as listed from Land Transport New Zealand website table http://vehicleinspection.nzta.govt.nz/virms/in-service-wof-and-cof/introduction/inspection-and-certification-process/identifying-the-vehicle-class: passenger vehicle MA, MB, MC; omnibus MD, MD1, MD2; and goods vehicle NA.

Service information may include but is not limited to – technical information of a vehicle, machine, or product detailing operation; installation and servicing procedures; manufacturer instructions and specifications; technical terms and descriptions; and detailed illustrations.  This may be accessed from the manufacturer.

Suitable tools and equipment refer to industry approved tools and equipment that are recognised within the industry as being the most suited to complete the task in a professional and competent manner with due regard to safe working practices.

5	Range

Assessment against this unit standard applies to painting a minimum of four major exterior panels on or from light or heavy vehicles in two pack solid colour.

6	Assessment

Evidence presented for assessment against this unit standard must be consistent with safe working practices and be in accordance with applicable manufacturer’s specifications, service information, company and legislative requirements (this includes the knowledge and/or use of suitable tools and equipment).

For this unit standard, it is essential that the practical assessment evidence is obtained in the workplace under normal workplace conditions.

7	Recommended skills and knowledge

Unit 24005, Use primers, fillers, and sealers, and prepare paint for top coats in the automotive refinishing industry; and Unit 24002, Mask an area for two pack painting in the automotive refinishing industry; or demonstrate equivalent knowledge and skills.

Outcomes and performance criteria

Outcome 1

Check the preparation of vehicle panels.

Performance criteria

1.1	Surface is checked for defects.

Range	sanding defects, repair defects, no missed sanding.

1.2	Masking is secure and smooth.

Range	no loose masking, remasking as necessary, all edges secure.

1.3	Surface is cleaned.

Outcome 2

Apply solid colour two pack coats on prepared vehicle panels.

Performance criteria

2.1	Suitable tools and equipment are identified and used.

Range	includes but is not limited to – air-fed respirator, gloves, dust free overalls, head covering; spray gun, spray booth, measuring stick, paint container, scales.

2.2	Spray gun is selected and set up in accordance with paint manufacturer’s specifications.

Range	may include but is not limited to – suction feed or gravity feed, equipment prepared, connections made, air pressure adjustment, fluid adjustment, fan adjustment.

2.3	Colour is identified and paint is prepared in accordance with paint manufacturer’s specifications.

Range	reducing and mixing ratios, colour mixing and matching, straining, measuring stick, paint container, scales.

2.4	Paint application requirements are identified and carried out.

Range	personal safety, safety of other people, vehicle safety, workshop safety, environmental safety, tools, equipment, spray booth, and machine safety, solvent, isocyanate and paint inhalation and absorption.

2.5	Paint is applied in accordance with paint manufacturer’s specifications.

2.6	Paint is dried in accordance with paint manufacturer’s specifications.

Range	force dried, chemical reaction.

surface is free of – dirt, runs, dryspray, heavy orange peel, sink back, uneven coating.

2.7	The surface is checked and minor defects are rectified.

Range	surface is free of – dirt, runs, dryspray, orange peel, sink back.

2.8	Tools and equipment are cleaned and put away, and the work area is cleaned.

2.9	Materials are sealed and stored.

| Planned review date | 31 December 2027 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 29 January 1996 | 31 December 2018 |
| Review | 2 | 20 December 1998 | 31 December 2018 |
| Revision | 3 | 16 October 2003 | 31 December 2018 |
| Review | 4 | 26 November 2007 | 31 December 2018 |
| Review | 5 | 21 April 2016 | 31 December 2027 |
| Review | 6 | 25 May 2023 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0014 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Hanga-Aro-Rau Manufacturing, Engineering and Logistics Workforce Development Council qualifications@hangaarorau.nz if you wish to suggest changes to the content of this unit standard.