| Title | Clean and sanitise food or related product production equipment manually |  |  |
| --- | --- | --- | --- |
| Level | 2 | Credits | 2 |

| Purpose | People credited with this unit standard are able to: prepare to clean and sanitise, clean, and sanitise food or related product production equipment manually. |
| --- | --- |

| Classification | Food and Related Products Processing > Food and Related Product Processing - Cleaning |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Legislation relevant to this unit standard may include but is not limited to the:
Health and Safety at Work Act 2015
Food Act 2014
Resource Management Act 1991.

2	Definitions

Cleaning and sanitising agents refer to items such as detergents, foams, caustic soda, chlorine based products.

Cleaning and sanitising equipment refers to items such as brushes, mops, brooms, cloths, sponges, sprays, fogging, vacuum cleaner.

Organisational procedures refer to documents that include worksite rules, codes, and practices; equipment operating instructions; manufacturer’s specifications; production specifications; documented quality management systems; and health and safety requirements, including the use of PPE.

PPE refers to personal protective equipment such as protective clothing, gloves, safety glasses, headwear, footwear, hearing protection, safety devices.

Production equipment refers to items such as packaging, food processing, mixing, blending, cooking, bowls, tables, conveyors, tools, vats, tanks, and surrounding work areas such as floors or walls.

Related products refer to beverages, household products, or personal care products.

3	Assessment information

All activities and evidence must be in accordance with organisational procedures.

Outcomes and performance criteria

Outcome 1

Prepare to clean and sanitise food or related product production equipment manually.

Performance criteria

1.1	Area and equipment to be cleaned and sanitised are prepared.

Range	may include but is not limited to – removal of debris.

1.2	Prior to scheduled cleaning, availability, and operability of cleaning and sanitising equipment are checked.

1.3	Cleaning and sanitising agents are checked for availability and for correct quantity and dilution for cleaning.

1.4	Production equipment is isolated from non-essential services and is checked for correct configuration for cleaning and sanitising.

Range	services may include but are not limited to – air, electricity, steam, water, gas.

Outcome 2

Clean food or related product production equipment manually.

Performance criteria

2.1	Production equipment is manually cleaned and left clean and free from contamination.

Range	may include but is not limited to – correct product(s), correct methods, minimal wastage of cleaning agent;

contamination may include but is not limited to – residual product, cleaning agents, foreign objects, microbiological.

2.2	Cleaning equipment is cleaned and stored.

Outcome 3

Sanitise food or related product production equipment manually.

Performance criteria

3.1	Production equipment is manually sanitised.

Range	may include but is not limited to – minimal wastage of sanitising agent.

3.2	Sanitising agent is applied in accordance with product specifications.

Range	may include but is not limited to – quantity, duration, safety.

3.3	Production equipment is sanitised and made free from microbiological contamination.

3.4	Sanitising equipment is cleaned and stored.

| Planned review date | 31 December 2025 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 27 August 1996 | 31 December 2022 |
| Revision | 2 | 15 May 1998 | 31 December 2022 |
| Review | 3 | 19 August 2004 | 31 December 2022 |
| Review | 4 | 19 March 2010 | 31 December 2022 |
| Review | 5 | 25 February 2021 | N/A |

| Consent and Moderation Requirements (CMR) reference | 0013 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact Competenz qualifications@competenz.org.nz if you wish to suggest changes to the content of this unit standard.