| Title | Demonstrate knowledge of tanker loading and safety equipment, and load product in an energy and chemical plant |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 3 |

| Purpose | People credited with this unit standard are able to: describe the function of energy and chemical tanker loading and safety equipment; prepare for loading product to a tanker; load product to a tanker and identify and rectify any deviations from normal operating conditions; and shutdown, document, and communicate the product loading process. |
| --- | --- |

| Classification | Energy and Chemical Plant > Operation of Energy and Chemical Plant |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Legislation relevant to this unit standard includes but is not limited to:

- Health and Safety at Work Act 2015;

- Health and Safety at Work (Hazardous Substances) Regulations 2017 (HSWA);

- Resource Management Act 1991;

- Land Transport Rule: Dangerous Goods 2005 (Rule 45001/1), available at http://www.nzta.govt.nz;

and any subsequent amendments.

2	Definitions

Energy and chemical plant may be in – petrochemical, agri-nutrient, power generation, dairy processing, meat processing, and wood fibre manufacturing, or other plants that operate with a combination of high temperatures, pressures, steam and/or chemicals in gas, liquid or solid form.

Organisational requirements – documented policies and procedures.  These may include: equipment manufacturers’ procedures; plant procedures; suppliers’ instructions; site signage; codes of practice; company health and safety plans; on site briefings; and supervisor’s instructions.  This includes all regulatory and legislative obligations that apply to the plant.

Plant – the operational unit, equipment and/or workplace at which the person is working.

Hazchem signage – the Hazchem emergency action code of numbers, letters and diamonds that give information to emergency services. Its use is required by the NZS 5433 PARTS 1 & 2:2012 Transport of dangerous goods on land: https://www.nzta.govt.nz/driver-licences/getting-a-licence/licences-by-vehicle-type/transporting-dangerous-or-hazardous-goods/dangerous-goods-carried-by-transport-operators.

- For the purposes of assessment:

- evidence for the practical components of this unit standard must be supplied from the workplace.

- evidence for all outcomes must be presented in accordance with organisational requirements.

Outcomes and performance criteria

Outcome 1

Describe the function of energy and chemical tanker loading and safety equipment.

Performance criteria

1.1	Describe tanker loading equipment and associated requirements in terms of their functions.

Range	loadout interface system, communication systems, control and shutdown systems, deadman alarm, additives, odorant, loading pumps, meters, printers, storage tanks, pipelines, hoses, loading arms, weigh bridges, manifolds, sampling points, drainage and bunding facilities, earthing and bonding facilities.

1.2	Describe safety equipment in terms of their function.

Range	signage, barriers, fire extinguishers, hoses, fire monitors, fire hydrants, foam systems, high flow systems, fire alarms, fire detectors, gas detectors, deluge systems, personal protective equipment, tanker master power switch, safety showers, eyewashes, spill kits, traffic control systems.

Outcome 2

Prepare for loading product to a tanker in an energy and chemical plant.

Performance criteria

2.1	Identify potential hazards in terms of their controls.

2.2	Check availability of product to load, documentation and/or scheduling.

Range	product available in storage, customer requirements.

2.3	Carry out preloading checks of tanker.

Range	checks include but are not limited to – tanker contents, tank capacity, tanker condition, product compatibility, odorization requirement, tanker positioning and connection, product quality, product quantity, earthing checks, Hazchem signage, NZ Transport Agency certification for prime mover; Ministry of Transport certification for tanker vessel, driver’s licence endorsement for dangerous goods, current site safety induction.

2.4	Carry out preloading checks of load-out facility.

Range	checks include but are not limited to – equipment availability, valve alignment, leaks, simultaneous loading operations, inter-tank transfers, product quality, product quantity, tank contents and condition.

2.5	Confirm safety equipment relating to product requirements as available and utilised.

Range	personal protective equipment, continuity tester, fixed fire protection, barriers, signage, emergency shutdown systems, gas detection systems, safety shower, eyewash facilities, spill kit, bunding, traffic control systems.

Outcome 3

Load product to a tanker and identify and rectify any deviations from normal operating conditions.

Performance criteria

3.1	Connect and start loading equipment.

Range	valving, tank and/or compartment capacity, level protection, pumps, compressors, flow control.

3.2	Carry out injection.

Range	additive, odorant.

3.3	Carry out product sampling.

Range	representative sample, double valve process isolation.

3.4	Control and monitor loading rates to ensure the product is loaded safely and within the tanker’s defined storage capacities.

3.5	Maintain communication with relevant parties to determine the progress of the loading operation, tank levels, and volumes.

Range	operator, driver, control room.

3.6	Identify any deviations from normal operating conditions and take actions to rectify them.

Range	vapour lock, pressure differential, operator error, spills, leaks, earth faults, slow loading flow, low odorization rate, excess flow, contamination, instrument malfunction, overfill, excessive venting.

Outcome 4

Shutdown, document, and communicate the product loading process.

Performance criteria

4.1	Verify the loading operation as complete and shut down equipment.

Range	isolation, vent down and slow loading arms, disconnection, leak check, disconnect tanker earthing.

4.2	Complete documentation and communicate the results of the loading to appropriate personnel.

Range	safety data sheets, dangerous goods declaration, bills of lading, load receipts, weighbridge documents, product certificate of quality, logbooks, computer records, control room log, operator log.

This unit standard is expiring.  Assessment against the standard must take place by the last date for assessment set out below.

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 6 February 1997 | 31 December 2018 |
| Revision | 2 | 3 August 2000 | 31 December 2018 |
| Review | 3 | 24 January 2002 | 31 December 2018 |
| Review | 4 | 20 February 2009 | 31 December 2018 |
| Rollover and Revision | 5 | 20 April 2017 | 31 December 2022 |
| Review | 6 | 27 February 2020 | 31 December 2026 |
| Review | 7 | 27 March 2025 | 31 December 2026 |

| Consent and Moderation Requirements (CMR) reference | 0079 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.