| Title | Demonstrate knowledge of and conduct pigging activities in an energy and chemical pipeline |  |  |
| --- | --- | --- | --- |
| Level | 3 | Credits | 3 |

| Purpose | People credited with this unit standard are able to: demonstrate knowledge of pigging, and load a pig into a launcher in a pipeline; launch a pig into a pipeline; receive a pig in a pipeline; and unload a pig from a pig receiver in a pipeline, reinstate the receiving system, and record and report findings. |
| --- | --- |

| Classification | Energy and Chemical Plant > Operation of Energy and Chemical Plant |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance Information

1	Legislation relevant to this unit standard includes but is not limited to:

- Health and Safety at Work Act 2015;

- Health & Safety at Work (Hazardous Substances) Regulations 2017 (HSWA);

- Resource Management Act 1991;

and any subsequent amendments.

2	Definitions

Energy and chemical plant may be in – petrochemical, agri-nutrient, power generation, dairy processing, meat processing, and wood fibre manufacturing, or other plants that operate with a combination of high temperatures, pressures, steam and/or chemicals in gas, liquid or solid form.

Organisational requirements – documented policies and procedures.  These may include: equipment manufacturers’ procedures; plant procedures; suppliers’ instructions; site signage; codes of practice; company health and safety plans; on site briefings; and supervisor’s instructions.  This includes all regulatory and legislative obligations that apply to the plant.

Plant – the operational unit, equipment and/or workplace at which the person is working.

Pig refers to pipeline internal gauges.  Pigs are devices for insertion into pipelines for the purpose of cleaning the lines or separating products.

- For the purposes of assessment:

- evidence for the practical components of this unit standard must be supplied from the workplace.

- evidence for all outcomes must be presented in accordance with organisational requirements.

- this unit standard relates to pipelines that carry liquid or gas petrochemicals.  It excludes the pigging of pipelines for other liquid or gas products.

Outcomes and performance criteria

Outcome 1

Demonstrate knowledge of pigging, and load a pig into a launcher in a pipeline.

Performance criteria

1.1	Describe the reasons for using pigs.

Range	cleaning, purging, product separation, clearing blockages, pipeline condition analysis.

1.2	Identify pig types in terms of application and site pigging requirements.

Range	displacement, bi-directional, intelligent, conical cup, gauging, batching (single or multiple), spherical, wire brush, scraper.

1.3	Identify hazards in relation to the interruption of pipeline flow.

Range	stuck pig, block valve closure, reduced flow rates, process interruption.

1.4	Load the pig and seat into neck of the launcher safely.

Range	launcher valves are confirmed closed, pressure vented from launcher, zero pressure confirmed, launcher door opened standing clear of open end, visual inspection of door seal.

Outcome 2

Launch a pig into a pipeline.

Performance criteria

2.1	Carry out pig launching system checks.

Range	visual inspection of threads and pressure bleed valves, pressure isolations, signalling/tracking equipment, quick opening enclosure, door closed, line valves are confirmed open, check for leaks.

2.2	Launch the pig into the pipeline safely.

2.3	Maintain communication relating to the process with relevant personnel prior to and during the pig launching operation.

Range	control room operator, operations and maintenance personnel, external customers.

2.4	Reinstate the pig launching system.

Outcome 3

Receive a pig in a pipeline.

Performance criteria

3.1	Carry out pig receiving system checks.

Range	visual inspection of threads and pressure bleed valves, pressure isolations, signalling/tracking equipment, quick opening enclosure, line valves are confirmed open, check for leaks.

3.2	Receive the pig and safely depressurise and drain the pig receiver.

3.3	Maintain communication relating to the process with relevant personnel during pig receiving operations.

Range	control room operator, operations and maintenance, external customers.

Outcome 4

Unload a pig from a pig receiver in a pipeline, reinstate the receiving system, and record and report findings.

Performance criteria

4.1	Carry out pig unloading system checks.

Range	pressure, isolations, catchment, signalling/tracking equipment, safety equipment.

4.2	Unload the pig safely and without spillage.

Range	receiver valves are confirmed closed, zero pressure confirmed, catchment equipment in place, receiver door opened standing clear of open end, visual inspection of door seal, pig unloaded and inspected, receiver door and vents/drains closed.

4.3	Reinstate the pig receiving system.

Range	receiver door seal cleaned and lubricated, receiver door closed, vents/drains closed, waste disposed of, check for leaks.

4.4	Record and report findings to relevant personnel.

Range	condition of pig, analysis of debris, reports, follow-up actions.

| Replacement information | This unit standard was replaced by skill standard 40388. |
| --- | --- |

This unit standard is expiring.  Assessment against the standard must take place by the last date for assessment set out below.

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 6 February 1997 | 31 December 2018 |
| Revision | 2 | 3 August 2000 | 31 December 2018 |
| Review | 3 | 24 January 2002 | 31 December 2018 |
| Review | 4 | 20 February 2009 | 31 December 2018 |
| Rollover and Revision | 5 | 20 April 2017 | 31 December 2022 |
| Review | 6 | 27 February 2020 | 31 December 2026 |
| Review | 7 | 27 March 2025 | 31 December 2026 |

| Consent and Moderation Requirements (CMR) reference | 0079 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.